3GPP TSG-SA3 Meeting #117	S3-243586

Maastricht, Netherlands 19 - 23 August 2024

# Source:	Ericsson

# Title:	Resolve the EN and update Solution#11

# Document for:	Approval

Agenda Item:	5.13

# 1	Decision/action requested

Approve the following changes to TR 33.784 [2].

# 2	References

[1]	3GPP TR 33.738 V18.0.1 Study on security aspects of enablers for network automation for the 5G system phase 3

[2]	3GPP TR 33.784 V0.2.0 Study on security aspects of core network enhanced support for Artificial Intelligence Machine Learning (AIML)

[3]             3GPP TR 23.700-84 V1.0.0 Study on Core Network Enhanced Support for Artificial Intelligence (AI) / Machine Learning (ML)

# 3	Rationale

This pCR tries to resolve the EN in Solution#11:

Editor's Note: Whether the conversion between internal UE ID to external UE ID is needed is FFS.

According to 3GPP TR 23.700-84 V1.0.1[3] clause 8.2 P#2.3.2, "the NEF may be involved in sample alignment with mapping of information (e.g. internal versus external information)", so the conversion between internal UE ID to external UE ID is needed. Thus this EN can be removed.

# 4	Detailed proposal

*** BEGIN CHANGES ***

## 6.11	Solution #11: Protection of Privacy of VFL between VFL members

### 6.11.1	Introduction

This solution addresses Key Issue #3 “Privacy of VFL between VFL members". As stated in the key issue description," The sample alignment procedure may involve the exchange of information (e.g. UE ID) which is sensitive and could potentially comprise the privacy of UEs."

This solution proposes that in order to protect the UE ID privacy, e.g. during VFL sampling procedure, NEF needs to perform the UE ID mapping from internal UE IDs (SUPI) to External UE IDs (AF specific UE Identifier) and vice versa, and then forwards VFL service request. According to TS 23.501 [11] clause 5.20, "The AF specific UE Identifier shall not correspond to a MSISDN; it is represented as a GPSI in the form of an External Identifier. When used as an AF specific UE identifier, the External Identifier provided by the 5GCN shall be different for different AF". So, the privacy of UE ID is protected.

### 6.11.2	Solution details

In order to protect the UE ID privacy, NEF performs the UE ID mapping from internal UE IDs (SUPI) to External UE IDs (AF specific UE Identifier) and vice versa, and then forwards VFL service request/response. VFL service request can be a service related to Sample alignment, feature spaces alignment or jointly participating to train an ML model.

Editor's Note: Whether the conversion between internal UE ID to external UE ID is needed is FFS.

#### 6.11.2.1	NWDAF acting as FL server

Figure 6.11.2.1-1:  Protection of Privacy of VFL between VFL members for NWDAF acting as FL server

Step 1. The NWDAF containing MTLF acting as VFL server sends the VFL service request/response to the NEF with Internal UE IDs, the Analytics ID information and the AF specific information.

Step 2. If internal UE IDs (SUPI) are received in step 1, the NEF performs internal UE IDs mapping to external UE IDs before forwarding the VFL service request.

Step 3a,3b. The NEF sends VFL service request to the Afs and receives the VFL service response. NWDAF and AFs will do the VFL preparation phase, sample and feature alignment.

Step 4. If external UE IDs are received in step 3, the NEF performs external UE IDs mapping to internal UI IDs (SUPI).

Step 5. The NEF forwards the VFL service response.

Step 6. The Vertical Federated Learning procedure is performed between FL server (NWDAF) and FL client (AF) via the NEF.

#### 6.11.2.2	External AF acting as FL server

Figure 6.11.2.2-1:  Protection of Privacy of VFL between VFL members for External AF acting as FL server

Step 1. The External AF acting as VFL server sends the VFL service request to the NEF with AF specific external UE IDs, the Analytics ID information and the NWDAF specific information.

Step 2. If AF specific external UE IDs are received in step 1, the NEF performs AF specific external UE IDs mapping to internal UE IDs i.e., SUPI before forwarding the VFL service request.

Step 3a,3b. The NEF sends VFL service request to the NWDAFs and receives the VFL service response. NWDAF and AFs will do the VFL preparation phase, sample and feature alignment.

Step 4. If internal UE IDs (SUPI) have been received in Step 3, the NEF performs mapping of internal UE IDs (SUPI) to external UE IDs.

Step 5. The NEF forwards the VFL Service Response.

Step 6. The Vertical Federated Learning procedure is performed between FL server (NWDAF) and FL client (AF) via the NEF.

### 6.11.3	Evaluation

Editor’s Note: Each solution should motivate how the potential security requirements of the key issues being addressed are fulfilled.

*** END CHANGES ***