3GPP TSG-SA3 Meeting #117																S3-243667

Maastricht, Netherlands  19 - 23 August 2024 					(revision of S3-242935)

# Source:	Nokia

# Title:	Discussion paper for Fast BSS details from 802.11 specs

# Document for:	discussion

Agenda Item:	5.8

# 1	Decision/action requested

It is requested to discuss the discussion paper on Fast BSS solution issue related to NSWO

# 2	References

[X]

# 3	Rationale

It was observed companies are not able to visualize the Fast BSS working defined in 802.11 specs. So in this we copying the relevant text from the 802.11 specs.

# 4	Detailed proposal

## 4.2 Basic Definition.

Fast BSS transition works within a Mobility Domain. According to IEEE 802.11-2020 specification:

- mobility domain: A set of basic service sets (BSSs), within the same extended service set (ESS), that support fast BSS transitions between themselves and that are identified by the set’s mobility domain identifier (MDID).

A Mobility Domain is an area within an ESS (extended service set).

- extended service set (ESS): A set of one or more interconnected basic service sets (BSSs) that appears as a single BSS to the logical link control (LLC) layer at any station (STA) associated with one of those BSSs.

Observation 1 It means, that Fast BSS Transition is restricted to a single Layer 2 broadcast domain.

Observation 2 It means that all connected UEs must be able to send an ARP to their default router or transmit a DHCP request to the server that had previously assigned their address.

Observation3: Even when FT is independent of IP connectivity issues, the specification requires that the IP connectivity can remain unchanged throughout the whole mobility domain.

From a specification standpoint, UEs stay connected to the ESS (bridged network) regardless of their IP address while performing fast transition handover. The network must ensure IP address preservation while UEs are moving.

## 4.2a TNGF domain and ESS domain.

There is no relation between TNGF domain and ESS domain. ESS domain is designed by Wi-Fi operators and TNGF domain is designed by telecom operator. So, it is possible that one TNGF1 domain may contains

- ESS1 domain

  - It contains TNAP/AP1a

  - IT contains TNAP/AP1b

- ESS2 domain.

  - It contains TNAP/AP2a

  - IT contains TNAP/AP2b

- TNAP/AP3

- TNAP/AP4

If UE is attached to TNAP/AP1 under ESS1, then

- FT will work when UE moves from TNAP/AP1a to TNAP/AP1b

- FT will not work when UE moves from TNAP/AP1a to TNAP/AP2b (ESS1 to ESS2)

- FT will work when UE moves from TNAP/AP1a to TNAP/AP3 (ESS1 to outside TNAP)

Observation: As companies are discussing and concluding only FT based solution, this limitation should clearly be marked in the TS. Or we should have some other solution agreed.

## 4.2 Call flow details of FT from 802.11

In this section, we copied the relevant text the 802.11 specs to understand the FT flow better

Chapter 13:

Fast BSS transition seeks to reduce the length of time that connectivity is lost between a STA and the DS during a BSS transition. The FT protocols are part of the reassociation service and only apply to STA transitions between APs within the same mobility domain within the same ESS.

The FT protocols require information to be exchanged during the initial association (or a later reassociation) between a STA [known as the FT Originator (FTO)] and AP. The initial exchange is referred to as the FTinitial mobility domain association. Subsequent reassociations to APs within the same mobility domain maymake use of the FT protocols.

APs advertise both capabilities and policies for supporting the FT protocols and methods.

13.2.2 Authenticator key holders

The R0KH and R1KH are responsible for the derivation of keys in the FT key hierarchy. For fast BSS transition, the functions of the IEEE 802.1X Authenticator are distributed among the R0KH and R1KHs. The R0KH interacts with the IEEE 802.1X Authenticator to receive the MSK resulting from an EAP authentication. The R1KH interacts with the IEEE 802.1X Authenticator to open the Controlled Port.  Both the R0KH and R1KH interactions with the IEEE 802.1X Authenticator occur within the SME.The R0KH derives the PMK-R0 for use in the mobility domain utilizing the MSK . The R0KH shall be responsible for deriving a PMK-R1 for each R1KH within the mobility domain.

Each R0KH-ID and R1KH-ID is assumed to be expressed as a unique identifier within the mobility domain. This identifier is communicated to the FTO and other key holders. The R0KH-ID is bound into the PMK-R0 derivation and the R1KH-ID is bound into the PMK-R1 derivation.

The R0KH and the R1KH are assumed to have a secure channel between them that can be used to exchange cryptographic keys without exposure to any intermediate parties. The protocol

for distribution of keying material from the R0KH to the R1KH is outside the scope of this standard.

13.3 Capability and policy advertisement

The FT capability is advertised in the Beacon and Probe Response frames by including the MDE. The MDE is advertised in the Beacon and Probe Response frames to indicate the MDID, FT capability, and the FT policy.

13.4.2 FT initial mobility domain association in an RSN

0 The FT capability is advertised in the Beacon and Probe Response frames by including the MDE. The MDE is advertised in the Beacon and Probe Response frames to indicate the MDID, FT capability, and the FT policy. (If an FTE is included in a Request element in a Probe Request frame, the FTE in the Probe Response frame shall contain the R0KH-ID and R1KH-ID (from dot11FTR0KeyHolderID and dot11FTR1KeyHolderID), and all other fields shall be set to 0.)

1,2 Upon successful IEEE 802.11 Open System or SAE authentication, if using a suite type for which the Authentication type column indicates FT authentication (see Table 9-151),

3: the STA shall send a(Re)Association Request frame to the AP that includes the MDE. The contents of the MDE shall be the values advertised by the AP in its Beacon or Probe Response frames. Additionally, the STA includes its security capabilities in the RSNE.

4: If the contents of the MDE received by the AP do not match the contents advertised in the Beacon and Probe Response frames, the AP shall reject the (Re)Association Request frame with status codeSTATUS_INVALID_MDE.

The (Re)Association Response frame from the AP shall contain an MDE, with contents as presented in Beacon and Probe Response frames. The FTE shall include the key holder identities of the AP, the R0KH-IDand R1KH-ID, set to the values of dot11FTR0KeyHolderID and dot11FTR1KeyHolderID, respectively.

FTE shall have a MIC element count of zero (i.e., no MIC present) and have ANonce, SNonce, and MIC fields set to 0.

5: Authentication: On successful (re)association, the S0KH on the STA and the R0KH on the AP then proceed with an IEEE 802.1X authentication using EAPOL PDUs carried in IEEE 802.11 Data frames if SAE authentication was not performed (

If IEEE 802.1X authentication was performed, then upon successful completion of authentication, the R0KH receives the MSK and authorization attributes.. If a key hierarchy already exists for this STA belonging to the same mobility domain (i.e., having the same MDID), the R0KH shall delete the existing PMK-R0 security association and PMK-R1 security associations. It then calculates the PMK-R0, PMKR0Name, and PMK-R1 and makes the PMK-R1 available to the R1KH of the AP with which the STA is associated.

If the MSK lifetime attribute is provided by the AS, the lifetime of the PMK-R0 shall not be more than the lifetime of the MSK. If the MSK lifetime attribute is not provided, the PMK-R0 lifetime shall be dot11FTR0KeyLifetime.

6,7,8,9: Four way handshake

Upon completion of a successful FT 4-way handshake, the IEEE 802.1X Controlled Port shall be opened on both the non-AP STA and the AP.

13.5.2 Over-the-air FT protocol authentication in an RSN

To perform an over-the-air fast BSS transition to a target AP, the FTO and target AP shall perform the following exchange:

FTOTarget AP: Authentication-Request (FTAA, 0, RSNE[PMKR0Name], MDE,

FTE[SNonce, R0KH-ID])

Target APFTO: Authentication-Response (FTAA, Status, RSNE[PMKR0Name], MDE,

FTE[ANonce, SNonce, R1KH-ID, R0KH-ID])

The SME of the FTO initiates the authentication exchange, through the use of the

MLME-AUTHENTICATE.request primitive, and the SME of the AP responds with an MLMEAUTHENTICATE. response primitive. See 11.3.4. The MLME primitives for Authentication when the FT authentication algorithm is selected use only authentication transaction sequence number values 1 and 2.

Observation 4 There are multiple kinds of FT procedures and AP/R0KH and STA/UE further exchanges nonce to generate FT keys.

# 5	Conclusions and proposals

Observation related to KIs/use cases of the TR:

- TNAP/TWAP Mobility (KI1 and KI3):

  - Fast BSS may work considering a wifi network is planned with proper ESS to cover multiple TNAPs/TWAPs.

  - If UE moves to an AP that does not come into the common mobility (e.g., part of ESS), then Fast BSS will not work, and UE goes for full primary authentication.

- AUN3 device/ RG mobility (KI2):

  - RGs are the cable network; RG works as a standalone AP to provide wifi connectivity to UE. For Fast BSS, UEs stay connected to the ESS (bridged network) via L2 while performing fast transition handover.

  - Between RGs, L2 layer connectivity is not defined, so it is questionable to have fast BSS among RGs.

  - If UE connections break from the RG1 and connect to RG2, then UE may invalidate the R0KH ID, so again, fast BSS will not work.

- NSWO (KI4):

  - ESS is a limited area where a wifi provider may provide common mobility. Secondly, for Fast BSS, UEs stay connected to the ESS (bridged network) via L2 while performing fast transition handover.

  - Currently, the NSWO can be providing to a UE connecting via AP from home country or visiting country. Using the Fast BSS for NSWO will be quite limited, and the solution will work only for a limited area where ESS is defined

Based on the observations made in this paper, fast BSS will work for TNAP mobility, but it may not work for RG and NSWO mobility. Therefore, for RG and NSWO mobility, non-Fast BSS based solution should be considered for normative work.