# 3GPP TSG-SA3 Meeting #118 	S3-243916

# October 14 –18, 2024  Hyderabad, IN

# Source: 	Interdigital

# Title: 	Alternatives to Solution #21

# Document for: 	Approval

Agenda Item: 	5.7

# 1	Decision/action requested

Approve the pCR to TR 33.700-29

# 2	References

[1]	TS 33.501 Security architecture and procedures for 5G System

[2]	TR 33.700-29 Study on Security Aspects of 5G Satellite Access 	in the 5G architecture

# 3	Rationale

This contribution proposes two alternatives to solution 21 aiming to remove the additional round trip and associated delay..

*** Start of 1st Change ***

6.S			Solution #S: Remediation of unauthenticated (D)DOS in S&F (Alternatives to Solution #21)

### 6.S.1			Introduction

### Solution #21 for the remediation of unauthenticated (D)DOS in S&F adds a round-trip and the associated delay. The following solutions as in Option 1 and Option 2 aim to remove the additional round-trip.

### 6.S.2.			Details

### 6.S.2.1			Alternative 1

This solution modifies Solution #21 and removes the round-trip with the associated delay.

Figure 6.S.2.1-1:  Remediation of unauthenticated (D)DOS in S&F (Alternatives to Solution #21 – Alternative 1)

0. The UE is provisioned with the Shared Secret, optional UE identity, and other parameters. This provisioning can be achieved out-of-band. This Shared Secret should be separate from the existing shared secret used for primary authentication.

1. The network equipment on board the satellite checks if the Feeder Link is available. If yes, the (D)DOS attack on availability is comparable to similar (D)DOS attacks in terrestrial networks, and the network equipment on board the satellite skips to step 11.

2. The network equipment on board the satellite decides to use puzzles to throttle/remediate a (D)DOS attack. The decision by the network equipment on board the satellite to use puzzles to throttle/remediate a DDOS attack could be based on the unavailability of the feeder link. This step may be skipped based on the results from step 1.

3. The network equipment on board the satellite prepares a puzzle. Composing puzzles may be delegated to other network entities and/or functions that may preconfigure the puzzle and evidence. This step may be skipped based on the results from step 1.

4. The network equipment on board the satellite offers (e.g., broadcasts)  the puzzle prepared in step 2. Optionally, a puzzle index may be transmitted here. Broadcasting shall ensure that the same puzzle is not being broadcast forever, the broadcast and puzzle strength shall be linked with the number of requests already handled/pending and network load. E.g. the complexity of the puzzle could increase to back off UnEn devices to introduce longer delays. Complexity can be linked with how much back off (delay) the network equipment on board the satellite wants to introduce for approaching UnEN devices. Optionally the network equipment on board the satellite could include the shared secret key identifier which can assist the UE to choose the shared secret key to derive the evidence. The UE could be provisioned with multiple shared secret keys (or a set of secret key parameters), each being identified by a secret key identifier. As broadcasting of puzzles requires computing power on the UE side to resolve, this could be a deterrent factor for deployment with low power/less complex/ambient power IoT devices or UEs. This step may be skipped based on the results from step 1.

5. The UE receives the broadcasted puzzle and solves it including the injection of the Shared Secret and optional UE identity and other pre-provisioned parameters and produces evidence. This step may be skipped based on the results from step 1. The injection of the pre-provisioned Shared Secret is done in this step by e.g., hashing the intermediate puzzle evidence with the Shared Secret. The intent of the injection is to make the evidence obtained from the common puzzle (i.e., received in the broadcast in step 3) be distinguishable per UE.

6. UE sends a Registration Request to the network equipment on board the satellite. This message includes evidence from step 5 and other optional parameters such as UE identity and the received puzzle corresponding to the one broadcasted in step 4. Note that instead of the actual puzzle received in step 4, a puzzle index, also optionally received in step 4 may be included in this step. This step may be skipped based on the results from step 1.

7. The network equipment on board the satellite receives the evidence sent from the UE in step 6 and saves it. Then it solves the puzzle similarly to step 4 including the injection of the Shared Secret and optional UE identity and other pre-provisioned parameters and produces the expected evidence. Optionally, the network equipment on board of satellite may check whether the solved puzzle or its index returned in step 6 corresponds to the broadcasted puzzle or its index from step 4. This step may be skipped based on the results from step 1. The injection of the pre-provisioned Shared Secret is done in this step by e.g., hashing the intermediate puzzle evidence with the Shared Secret. The intent of the injection is to make the evidence obtained from the common puzzle be distinguishable per UE.

8. The network equipment on board the satellite verifies the evidence received from the UE by comparing it with the evidence produced in step 6. If successful, the procedure follows with step 9, and if not step 8. This step may be skipped based on the results from step 1.

9. The network equipment on board the satellite sends a Registration Reject to the UE. This step may be skipped based on the results from step 1.

10. Satellite detects feeder link availability. This step may be skipped based on the results from step 1.

11. The network equipment on board the satellite forwards the Registration Request over the feeder link to the terrestrial network elements.

12. The registration procedure continues as per TS 33.501 Figure 4.2.2.2-1: Registration Procedure.

### 6.S.2.2			Alternative 2

This solution modifies Solution #21 and removes the round-trip with the associated delay.

Figure 6.S.2.2-1:  Remediation of unauthenticated (D)DOS in S&F (Alternatives to Solution #21 – Alternative 2)

The steps in Figure 6.S.2.2.-1 are described below.

0. The UE is provisioned with the Shared Secret and optional UE identity and other parameters. This provisioning can be achieved out-of-band. This Shared Secret should be separate from the existing shared secret used for primary authentication.

1. The network equipment on board the satellite checks if the Feeder Link is available. If yes, the (D)DOS attack on availability is comparable to similar (D)DOS attacks in terrestrial networks, and the network equipment on board the satellite skips to step 11.

2. The network equipment on board the satellite decides to use puzzles to throttle/remediate a (D)DOS attack. The decision by the network equipment on board the satellite to use puzzles to throttle/remediate a DDOS attack could be based on the unavailability of the feeder link. This step may be skipped based on the results from step 1.

3. The network equipment on board the satellite prepares a puzzle. Composing puzzles may be delegated to other network entities and/or functions that may preconfigure the puzzle and evidence. This step may be skipped based on the results from step 1.

4. The network equipment on board the satellite solves the puzzle including the injection of the Shared Secret and optional UE identity and other pre-provisioned parameters and produces the expected evidence. This step may be skipped based on the results from step 1. The injection of the pre-provisioned Shared Secret is done in this step by e.g., hashing the intermediate puzzle evidence with the Shared Secret. The expected evidence from this step is saved. The injection intends to make the evidence obtained from the common puzzle be distinguishable per UE.

5. The network equipment on board the satellite offers (e.g., broadcasts)  the puzzle prepared in step 2. Optionally, a puzzle index may be transmitted here. Broadcasting shall ensure that the same puzzle is not being broadcast forever, the broadcast and puzzle strength shall be linked with the number of requests already handled/pending and network load. E.g. the complexity of the puzzle could increase to back off UnEn devices to introduce longer delays. Complexity can be linked with how much back off (delay) the network equipment on board the satellite wants to introduce for approaching UnEN devices. Optionally the network equipment on board the satellite could include the shared secret key identifier which can assist the UE to choose the shared secret key to derive the evidence. The UE could be provisioned with multiple shared secret keys (or a set of secret key parameters), each being identified by a secret key identifier. As broadcasting of puzzles requires computer power on the UE side to resolve them, this could be a deterrent factor for the deployment with low power/less complex/ambient power IoT devices or UEs. This step may be skipped based on the results from step 1.

6. The UE receives the broadcasted puzzle and solves the received puzzle including the injection of the Shared Secret and optional UE identity and other pre-provisioned parameters and produces evidence. This step may be skipped based on the results from step 1. The injection of the pre-provisioned Shared Secret is done in this step by e.g., hashing the intermediate puzzle evidence with the Shared Secret. The injection intends to make the evidence obtained from the common puzzle (i.e., received in the broadcast in step 3) distinguishable per UE.

7. UE sends a Registration Request to the network equipment on board the satellite. This message includes evidence from step 4 and other optional parameters such as UE identity and the received puzzle corresponding to the one broadcasted in step 5. Note that instead of the actual puzzle received in step 4, a puzzle index, also optionally received in step 5 may be included in this step. This step may be skipped based on the results from step 1.

8. The network equipment on board the satellite receives the evidence sent in step 7 and verifies the evidence received from the UE by comparing it with the evidence produced in step 6. If successful, the procedure follows with step 10, and if not step 9. Optionally, the network equipment on board of satellite may check whether the solved puzzle or its index returned in step 7 corresponds to the broadcasted puzzle or its index from step 5. This step may be skipped based on the results from step 1.

9. The network equipment on board the satellite sends a Registration Reject to the UE. This step may be skipped based on the results from step 1.

10. Satellite detects feeder link availability. This step may be skipped based on the results from step 1.

11. The network equipment on board the satellite forwards the Registration Request over the feeder link to the terrestrial network elements.

12. The registration procedure continues as per TS 33.501 Figure 4.2.2.2-1: Registration Procedure.

### 6.S.3			Evaluation

Both proposed solutions address the requirement of Key Issue #1 and do not require pre-provisioning the UE with satellite-specific parameters.

Further evaluation is FFS.

*** End of 1st Change ***