3GPP TSG-SA3 Meeting #118	S3-243930

Hyderabad, India, 14th – 18th October 2024

# Source:	MITRE-FFRDC

# Title:	eZTS Sol 11 Evaluation

# Document for:	Approval

Agenda Item:	5.1

# 1	Decision/action requested

This pCR proposes evaluations for Solution #11 in TR 33.794[1].

# 2	References

[1]	3GPP TR 33.794 Study on enablers for Zero Trust Security

# 3	Rationale

This document proposes evaluations for Solution #11 in TR 33.794 [1]. Solution #11 provides 5GS procedures to dynamically provision security policies, thus enabling dynamic policy enforcement.

# 4	Detailed proposal

SA3 is kindly requested to approve the below change to TR 33.794 [1]

********************** Start of 1st Change ****************************

## 7.11	Solution #11: Dynamic Security Policy Enforcement Framework

### 7.11.1	Introduction

This solution addresses KI#2 (Key Issue #2: Security mechanisms for policy enforcement at the 5G SBA); Specifically, it addresses Security policy enforcement Use Case #1: Access control decision enhancement (i.e., how the data from security monitoring can be considered in access decisions).

This solution defines two security policy enforcement frameworks (i.e., Indirect and Direct Policy Enforcement) able to enforce a dynamic security policy.

The dynamic security policy includes dynamic authorization policies such as those defined in the NF profiles (e.g.,  allowedRuleSet) but with enhancements that include expiration times and policy management capabilities. This additional dynamic security policy is used to supplement the authorization framework defined in TS 33.501 [3] clause 13.4.1 and does not rely on the NF profile update with the NRF. The dynamic security policy can be used as a containment measure during ongoing security incidents to reduce further compromise. For example, the dynamic authorization policies can be used to temporarily employ a directive of minimal functionality across all the NFs during a DoS attack on the PLMN core.

#### 7.11.1.1 Indirect Policy Enforcement

For indirect communication, authorization between the NF and the SCP is already done via local authorization policy in the SCP, as stated in TS 33.501 clause 13.3.6 “Authorization between the SCP and NFs is based on local authorization policy”. This solution proposes to enhance these local authorization policies with a dynamic security policy that can be updated by the operator. Similarly, this use of dynamic security policy is proposed as an additional layer of authorization policy for NFs.

#### 7.11.1.2 Direct Policy Enforcement

For direct communications, the dynamic security policy can be used as a PLMN-wide security policy (e.g. NF instance ID XYZ declared not trusted). The dynamic security policy can be provided by the operator.  The dynamic security policy is applied to NFs, including the NRF when deciding to issue an access token.

### 7.11.2	Solution details

Figure 7.11.2-1: Security Policy Enforcement Framework

- NF and/or SCP subscribe to, or query (e.g., on a per NF/NF communication basis) the security policy updates from the Security Policy Distribution Function (e.g., NRF may take the role of Security Policy Distribution Function).

- The Operator Security Function (OSF) sends security policy updates to Security Policy Distribution Function when there are changes in the security policy of one or more NFs. An example of a dynamic security policies is provided in clause 7.11.2.1.

NOTE 1: The triggering actions for sending and updated security policy is up to the operator.

- Indirect Policy Enforcement: Security Policy Distribution Function sends the security policy update to an SCP via SBI. The SCP updates the dynamic authorization policy accordingly and applies it to any ongoing or future SBA communication to enable Indirect Policy Enforcement.

- Direct Policy Enforcement: Security Policy Distribution Function sends the security policy update to the NF Service Producer or Consumer via SBI. NF service consumer or producer updates the dynamic authorization policy accordingly and applies it to any ongoing or future SBA communication to enable Direct Policy Enforcement.

#### 7.11.2.1	Dynamic Security Policy details

The table 7.11.3-1 below provides example policies for the dynamic authorization policies described in this solution. The Attributes can be taken from the NF profile authorization policies defined in TS 29.510 (e.g., RuleSet, snpns, plmns, nfTypes). The Policy ID is a unique identifier for the security policy and used for management of the policy. Distribution defines the recipients of the updated security policy. The Policy Action describe the required action (ALLOW/DENY) to be taken by the distribution entity and applied to incoming messages matching the attributes. The validity attribute defines the expiration time of this policy.

Table 7.11.2.1-1: Example Dynamic Security Policy

| Policy ID | Distribution | Attributes | Policy Action | Validity |
| --- | --- | --- | --- | --- |
| 1 | NRF | scopes: serviceX nfInstances: [NFInstanceA, NFInstanceB] | DENY | 2024-09-28T23:00:00.0000000Z |
| 2 | ALL | plmns: [plmnA, plmnB] | ALLOW | 2024-09-28T23:04:00.0000000Z |
| 3 | SMF, AMF | nfTypes: UPF | DENY | 2024-09-28T23:05:00.0000000Z |
| 4 | UDM (instance ID) | nfTypes: AMF | DENY | 2024-09-28T23:06:00.0000000Z |

### 7.11.3	Evaluation

This solution addresses the security threat of KI #2 by providing dynamic security policies to the NRF to mitigate the threat of a potentially compromised NF.

This solution defines a dynamic security policy enforcement framework with a mechanism to deliver security policy (i.e., Indirect and Direct Policy Enforcement).

There may be impacts to existing Security Policy Distribution Function (e.g., NRF) services including subscribe/notify and query/response services for dynamic security policy updates.

This solution brings enhanced 5GS functionality to support a dynamic authorization policy to enable dynamic security policy enforcement.

********************** End of 1st Change ****************************