3GPP TSG-SA3 Meeting #118	S3-243932

Hyderabad, India, 14th – 18th October 2024

# Source:	MITRE-FFRDC, OTD_US

# Title:	eZTS KI 2 Conclusions

# Document for:	Approval

Agenda Item:	5.1

# 1	Decision/action requested

This pCR proposes conclusions for TR 33.794[1] to specify requirements for dynamic secrurity policies.

# 2	References

[1]	3GPP TR 33.794 Study on enablers for Zero Trust Security

[2]	3GPP TS 29.510 5G System; Network function repository services; Stage 3

# 3	Rationale

The present document captures the dynamic security policies requirements that are to be part of the conclusion. The policies are to be related to existing capabilities in 3GPP TS 29.510 [2].

# 4	Detailed proposal

SA3 is kindly requested to approve the below change to TR 33.794 [1]

********************** Start of 1st Change ****************************

# 8	Conclusions

Editor’s Note: This clause contains the agreed conclusions that will form the basis for any normative work.

## 8.X Dynamic security policies

### 8.X.1 Security requirements

The following system level security requirements address KI#2 Security mechanisms for policy enforcement at the 5G SBA layer.

General requirements for dynamic security policies support:

- The 5G system supports the receipt of dynamic security policies from the OSF.

- The NRF supports dynamic security policy configuration from the OSF.

- The NRF supports the ability to distribute dynamic security policies to NFs and the SCP.

- The NRF and NF support the secure storage and management of dynamic security policies.

- The 5G system only allows the dynamic security policies to be managed by an authenticated and authorised entity.

The following requirements are related to configuration of dynamic security policies:

- A dynamic security policy can be applicable to all NFs, one or more NF types (e.g. AMF, SMF), one or more unique NFs, or a combination of one or more types and one or more unique NFs.

- A dynamic security policy can have one or more attributes associated with it. The attributes define when a policy is applicable.

- A dynamic security policy includes a policy action (i.e., allow, deny).

- A dynamic security policy includes a validity time.

The following requirement is related to dynamic security policy attributes format:

- Dynamic security policy attributes follow the format of the AllowedRuleSet IE defined in TS 29.510 [20].

********************** End of 1st Change ****************************