3GPP TSG-SA3 Meeting #118	S3-244011

Hyderabad, India, October 14 – 18, 2024

# Source:	OPPO

# Title:	Evaluation for AIoT Authentication Based on 5G-AKA

# Document for:	Approval

Agenda Item:	5.9

1	Decision/action requested

Approve the pCR to TR 33.713

2	References

[x]	TS 33.501 Security architecture and procedures for 5G System

[y]	TR 38.769 Study on solutions for Ambient IoT (Internet of Things) in NR

[z]	TS 33.102 3G security; Security architecture

[xx]	TS 35.205 Specification of the MILENAGE algorithm set: An example algorithm set for the 3GPP authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5*; Document 1: General

[yy]	R2-2406202 RAN2#126 Meeting Report

[zz] 	"Novel Authentication Protocols Tailored for Ambient IoT Devices in 3GPP 5G Networks" available at https://arxiv.org/pdf/2404.02425.pdf.

3	Rationale

This contribution proposes a 5G-AKA[x] based authentication solution for Key Issue #5.

According to the following scopes in TR 38.769[y], there are constrains such as power consumption etc.

The overall objective of the SI is to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices:

i.	~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally.

ii.	≤ a few hundred µW peak power consumption has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally.

For 5G-AKA, the generation of AV uses functions f1~f5 described in 3GPP TS 33.102.

f1：Message authentication function used to compute MAC

f2：Message authentication function used to compute RES and XRES

f3：Key generating function used to compute CK

f4：Key generating function used to compute IK

f5：Key generating function used to compute AK in normal procedure

For R19 Ambient IoT device, it’s unlikely able to support so many calculations due to the power constrain. So, a lightweight authentication solution for Ambient IoT service is proposed with the following evaluation.

4	Detailed proposal

This proposal aims to provide evaluation for AIoT Authentication solution based on 5G-AKA and resolve the editor notes in the authentication KI#5. SA3 is suggested to approve the following pCR.

*** Start of 1st Change ***

6.X			Solution #X: AIoT Authentication solution based on 5G-AKA

6.X.3			Evaluation

Device authenticating network is mandatory procedure for the “inventory and command” case [yy], to make sure the step C “reader to device data transmission” is from the legitimate network element.

For the “inventory and command” case in step D, wether network authenticating device is optional is FFS, since Step D “corresponding device to reader data transmission” is FFS whether this is optional for RAN2 [yy].

AIoT Device securely stores the root key K.

This solution uses less functions comparing to 5G AKA[x] [z] during authentication, thus it’ a lightweight solution more suitable for power constrain of AIoT device. The power constrain of AIoT device is described in TR 38.769[y].

For 5G AKA, in order to device XMAC/RES at the AIoT device side, 3 functions (f1, f2, f5) are needed.

For lightweight AIoT authentication solution, in order to device MAC/XRES at the network side, only 2 functions are needed. If lightweight functions such as AES is used for these functions [xx], it will further reduce the power consumption [zz].

*** End of 1st Change ***

2		References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.369: "Service Requirements for ambient power-enabled IoT".

[3]	RP-244058, RAN New SID for Study on Solution for Ambient IoT in NR.

[4]	3GPP TR 23-700-13: "Study on Architecture Support of Ambient power-enabled Internet of Things".

[x]	TS 33.501 Security architecture and procedures for 5G System

[y]	TR 38.769 Study on solutions for Ambient IoT (Internet of Things) in NR

[z]	TS 33.102 3G security; Security architecture

[xx]	TS 35.205 Specification of the MILENAGE algorithm set: An example algorithm set for the 3GPP authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5*; Document 1: General

[yy]	R2-2406202 RAN2#126 Meeting Report

*** End of 2nd Change ***