3GPP TSG-SA3 Meeting #118	S3-244025

Hyderabad, India  14 – 18 October 2024

# Source:	Nokia

# Title:	KI2 Solution on Authorization token request handling in CAPIF interconnect

# Document for:	Approval, Information, Discussion

Agenda Item:	5.19

# 1	Decision/action requested

Solution on authorization aspect in CAPIF interconnect

# 2	References

[1]	3GPP TS 23.700-22

[2]	3GPP TS 33.700-22

# 3	Rationale

Solution on authorization aspect in CAPIF interconnect to address KI2

# 4	Detailed proposal

**********   START OF CHANGES

## B.X	Solution #X: Authorization token request handling in CAPIF interconnect

### B.X.1	Introduction

This solution is addressing KI#2 on security aspects for CAPIF interconnect, specifically the authorization aspect that an access token needs to be requested by the CCF from the other CCF to which the AEF is associated.

### B.X.2	Solution details

#### B.X.2.1 Summary

An API invoker is onboarded at CCF-B, i.e. in the same trust domain, and wants to consume application services offered by an AEF outside of its trust domain. The AEF services are therefore not registered at the same CCF (CCF-B) as the API invoker but at a CCF-A. Hence, in interconnect an access token needs to be requested from CCF-A in a different trust domain. This is done by the CCF-B where the API invoker belongs to.

The API invoker includes onboarding secret and possibly also a client credential assertion (CCA) information into its access token request. CCF-B processes the information and forwards to CCF-A. Note, the onboard secret is not needed at CCF-A, hence it will be removed.

CCF-A verifies the access token request and CCA, if available. If successfully verified, CCF-A provides back to CCF-B an access token, with the client identifier in the access token claims set to the source API invoker ID as verified before.

Note: The assumption is that cross-domain certification is enabled, which allows CCF-A to verify the signature of the requesting API invoker known in CCF-B before creating the access token.

CCF-B provides the access token to the API invoker, which then can establish a TLS connection with AEF and invoke the northbound API with an OAuth 2.0 Access Token.

#### B.X.2.2 Information flow

Figure B.X.2.2-1: Information flow to retrieve security method in interconnect

Step 1: CAPIF-1/e authentication and secure session establishment is performed.

NOTE 0: APIInvoker can be residing on a UE or can be any outside the UE (e.g., an Application Function).

Step 2: After successful establishment of TLS session over CAPIF-1e, the API invoker sends to the CCF-B an Access Token Request message and optionally CCA token signed (using the APIInvoker’s private key) as per the OAuth 2.0 specification.

NOTE 1: The API invoker may include the CAPIF core function assigned API invoker ID and the Onboard_Secret in the OAuth access token request message for the CAPIF core function to validate the access token request.

NOTE 2: The CCA token will be compliant to Json Web Token IETC RFC 7519 and includes the APIInvokerID, CCF ID, timestamp (iat) and the expiry time (exp). The lifetime of the CCA token is expected to be smaller than the expiry time associated with the OAuth 2.0 access token. The CCA token when received at CCF-A ensures the request is originated by APIInvoker.

Step 3: CCF-B verifies the Access Token Request message per OAuth 2.0 specification. In the above figure, Resource Owner is shown as part of CCF-A’s domain. Instead, if Resource Owner is part of CCF-B’s domain, then CCF-B can fetch the resource owner consent using RNAA (e.g. if consent is the applicable legal basis).

Step 4: If CCF-B cannot successfully verify the Access Token Request message from APIInvoker, it provies an error message back.

If successfully verified, CCF-B creates a new Access Token request using the Access Token Request it received from APIInvoker. The new Access Token Request does not include the Onboard_Secret as received in the access token request in step 2) anymore. The remaining parameters in step 2 are reused. In addition, the source, e.g.  "sourceAPIInvokerID” is added and is set to the value of client_id received in step 2 (APIinvokerID). client_id is set to CCF-B identifier.

If the Resource Owner is part of CCF-B’s domain then and consent was retrieved, then CCF-B can also include the consent information.

If the Resource Owner is within CCF-A’s domain, and the consent needs to be captured, then CCF-B includes information about the APIInvoker identity (sourceAPIInvokerID) as well as consent-specific parameters (e.g., purpose of the data processing). By this, the RO can identify the party requesting access to the protected resources and the reason for that.  The APIInvoker certificate, which is retrieved locally, is added as additional IE.

Step 5: CCF-B sends the newly generated OAuth access token request to CCF-A along with the CCA token, if available and as provided by the APIInvoker) and the APIInvoker certificate.

Step 6: CCF-A verifies the Access Token Request as per OAuth2.0 specification. (more details are below)

Step 7: CCF-A verifies if CCF-B is authorized for the service. It validates the CCA token, if available, with the received APIInvoker certificate. It validates whether the sourceAPIInvokerID and CCF-B id in CCA token are matching with the Access Token Request received. CCA token verification ensures the Access Token Request is originated from APIInvoker and also to authenticate APIInvoker at CCF-A.

CCF-A verifies if the APIInvoker is authorized for the service if RO is part of CCF-B and consent information if available in the token is valid, or if RO is part of CCF-A’s domain, and consent is applicable. In the latter case, it gets the consent from resource owner using RNAA.

Step 8: After successful validation, CCF-A generates an access token response with a token including client_id in AccessTokenClaims set to sourceAPIInvokerID present in step 4.

Step 9: CCF-A sends the access token response to CCF-B.

Step 10: CCF-B forwards the access token response to APIInvoker.

Step 11-12: After successful authentication to the AEF on CAPIF-2e, the API invoker shall initiate invocation of a 3GPP northbound API with the AEF. The access token received in step 10 is included along with the northbound API invocation request.

Step 13: The API exposing function shall validate the access token. The AEF verifies the integrity of the access token by verifying the CAPIF core function signature. If validation of the access token is successful, the AEF verifies the API invoker's Northbound API invocation request against the authorization claims in access token, ensuring that the API Invoker has access permission for the requested service API.

Step 14: After successful verification of the access token and authorization claims of the API invoker, the requested northbound API is invoked, and the appropriate response is returned to the API invoker.

### B.X.3	Evaluation

The solution addresses KI#2 aspects of requesting an OAuth token.

Since the API invoker will always requet its own CCF for the access token, in interconnect scenarios CCF needs additional processing because the OAuth access token is issued by the CCF where the AEF is registered. In summary, onboard information is not needed, CCF id as the Aouth client and API invoker information needs to be added to allow for access token creation. To allow the CCF in the other domain to verify the API invoker information a CCA token signed (using the APIInvoker’s private key) as per the OAuth 2.0 specification can be added.

**********   END OF CHANGES