3GPP TSG-SA3 Meeting #118	S3-244129

Hyderabad, India 14 - 18 October 2024

# Source:	Qualcomm Incorporated, Huawei, Hisilicon, Cablelabs

# Title:	Discussion on open issues of using FT under a TNGF

# Document for:	Approval

Agenda Item:	4.13

# 1	Decision/action requested

This contribution discusses some of the open issue related to mobility under a TNGF

# 2	References

None

# 3	Rationale

This contribution discussed some of the open issues from the discussion on the text for using FT under a TNGF.

New key or re-use of KTNAP

The use of FT as described in the IEEE 802.11 when using a 256-bit key to create the FT keys relies on 512 bits of keying material to be sent. One 256-bit part of this key material is used as the MPMK at the ROKH to generate the FT keys and the other part is used a PMK at the initial AP access. This means that KTNAP cannot be used to support FT.

Furthermore using KTNAP for both purposes would be against the security principles of the 5G key hierarchy where each derived key should have one use, i.e. KTNAP is used a PMK and therefore should not be used a the R0KH.

Hence it is proposed to use a new key.

Location of R0KH

From a 5G network perspective, the IPsec that terminates at the TNGF protects the NAS signalling and user plane. Therefore the location of the R0KH does not affect the security of the access to the 5G network. Furthermore, the APs in the TNAN may be used for non-3GPP purposes and for that would not require a R0KH at the TNGF. Hence trying to force the R0KH to be deployed at a particular location would overly restrict the deployment of the TNAN. Finally, the architecture inside the TNAN is not in 3GPP scope and hence the location of the R0KH is not in 3GPP scope.

Hence it is proposed not to restrict the location of the R0KH.

TNAP support of FT

The list of TNAPs that support FT can be configured at the TNGF and hence there is no need to signal support between the TNAP and TNGF. As noted above the internal details of the TNAN are not in 3GPP scope and hence this issue can be left to the TNAN deployment.

Hence it is proposed to add a NOTE to leave this to the TNAN deployment.

Deployment of FT capable networks under a TNGF

3GPP standards define logical elements and the interaction between them. There is also no need to have any relationship between the MDID used in FT and the TNGF identity. Hence it is possible to have one or more complete FT mobility domains identified by different MDIDs deployed in a TNAN.

Hence it is proposed to include a NOTE to capture these possible deployments.

# 4	Detailed proposal

It is proposed to use the above discussion point when discussing using FT under a TNGF.