3GPP TSG-SA3 Meeting #118	S3-244134

Hyderabad, India 14 - 18 October 2024

# Source:	Qualcomm Incorporated

# Title:	Proposed conclusions for key issues #1 & 2 for HSS in satellite

# Document for:	Approval

Agenda Item:	5.7

# 1	Decision/action requested

This contribution proposes a conclusion for the Satellite Store & Forward use case when the HSS is in the satellite.

# 2	References

[1]	S3-243555 Draft TR 33.700-29 v0.4.0

[2]	S3-243533 LS on reply to LS on FS_5GSAT_Ph3_ARCH conclusions

# 3	Rationale

SA2 have concluded on two deployments for the satellite store & forward use. The first case is having the HSS deployed in the satellite. In such a case, there is no need for new security procedures and also no need for additional changes to preserve privacy as the access is essentially the same as regular LTE.

One difference is that due to the need for a key in the satellite to generate AVs. For this issue, an IOPS based solution could be deployed (note: whether additional are made to the IOPS solution for satellite can be left to the normative phase). The use of IOPS like solution would be in scop of the operator as all the functionality is in the USIM and HSS.

Finally the interfaces from the HPLM to the satellite is out of scope for SA2 and hence should be left out scope for SA3.

Such conclusions are aligned with the LS [2] sent to SA2 from the last SA3 meeting.

# 4	Detailed proposal

It is proposed to include the below pCR in the draft TR [1].

**** START OF CHANGES ****

# 7	Conclusions

## 7.1	Key Issue #1: Security protection in Store and Forward Satellite Operation

For the HSS on the satellite deployment, the following conclusions are made:

The normal security procedures are used as security can be established using legacy security procedures using the security credentials in the HSS onboard the satellite;

An (enhanced) IOPS based solution (see Annex F in TS 33.401 [3]) can be used to limit the impact of exposing the long term key in the HSS in the satellite as described in Solution #3; and

NOTE: Any enhancements of IOPS are left to the normative phase.

the security of communications between the proxies on satellite and ground is out of 3GPP scope.

## 7.2	Key Issue #2: Key Issue on privacy threats in S&F operation

For the HSS on the satellite deployment, no normative work is required.

**** END OF CHANGES ****