3GPP TSG-SA3 Meeting #118	S3-244166

Hyderabad, India  14 – 18 October 2024

# Source:	Ericsson

# Title:	Solution for key issue #1.2

# Document for:	Approval

Agenda Item:	5.19

# 1	Decision/action requested

It is proposed to add the following solution to the TR 33.700-22.

# 2	References

[1]	3GPP TR 33.700-22: "Study on security aspects of CAPIF Phase3".

# 3	Rationale

This document introduces a solution to TR 33.700-22 [1] for key issue #1.2 (Resource owner authorization management).

# 4	Detailed proposal

It is proposed to approve the following change to TR 33.700-22 [1].

***START OF CHANGES***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[3]	3GPP TR 23.700-22: "Study on CAPIF Phase 3".

[4]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[XX]	OpenID Connect Client-Initiated Backchannel Authentication Flow 1.0.

***NEXT CHANGE***

### 6.X	Solution #X: Client initiated backchannel authorization (CIBA) based solution

#### 6.X.1 	Introduction

This solution addresses the requirements identified in key issue #1.2 (Resource owner authorization management).

In some cases, there can be no preconfigured authorization information and the permission from the resource owner is requested by an out of band mechanism. Client credentials and authorization code flow (including PKCE) are not enough to address these type of use cases. Utilization of client initiated backchannel authorization (CIBA) [XX] flow is proposed for such use cases.

#### 6.X.2	Solution details

High-level steps of the solution are presented in Figure 6.X.2-1 and explained below.

Figure 6.X.2-1: High-level procedure flow of the solution

1.	The AF sends the authorization request to the CCF.

2.	The CCF checks whether the request requires resource owner permission. If the request requires resource owner permission and there is no available resource owner permission data, then the CCF decides to use CIBA flow.

3.	The CCF informs the AF about initiating the CIBA flow, so that the AF executes steps 5 and 6 regularly until the CCF obtains permission from the resource owner.

4.	The CCF obtains permission from the resource owner by using an out of band mechanism.

5-6. The AF regularly sends the token request to the CCF and CCF sends the response which depends on whether the CCF has obtained the permission from the resource owner.

#### 6.X.3	Evaluation

The solution fills the missing part in the CAPIF RNAA mechanism by addressing the case that the resource owner is not online in the authorization flow and there is no resource owner permission in advance. Thus, the solution mainly addresses the availability expectation which is one of the pillars in the CIA security triad.

***END OF CHANGES***