3GPP TSG-SA3 Meeting #118	S3-244174

Hyderabad, India  14 – 18 October 2024

# Source:	Ericsson

# Title:	Update for solution 5

# Document for:	Approval

Agenda Item:	5.11

# 1	Decision/action requested

Approve the pCR to TR 33.759 [1].

# 2	References

[1]	3GPP TR 33.759: "Study on security enhancements of Uncrewed Aerial Systems (UAS) Phase 3"

# 3	Rationale

This contribution proposes an update for the solution 5 in the UAS3 TR 33.759.

# 4	Detailed proposal

*****Start of changes*****

## 6.5	Solution #5: UUAA and authorization of target USS for the multiple USS case

### 6.5.1	Introduction

This solution addresses key issue #1 (security enhancements to NEF services in support of multiple USSs) by providing enhancements to the UUAA and authorization of target USS in invoking exposure services for the UAV.

### 6.5.2	Solution details

#### 6.5.2.1	Identification of USS-level changeover need

For identification of changeover need, USS-A can use a planned flight path of the UAV, some data analytics to predict the location of the UAV, use the information of intersection of geographical areas served by both USSes, or can rely on notifications from the UAS NF.

The changeover procedure includes the authentication and authorization (A&A) procedure executed between the UAV and the target USS, so changeover can be completed successfully after the successful A&A procedure.

#### 6.5.2.2	Enhancement for UUAA

When the serving USS (USS-A) that has performed the UUAA procedure with the UAV identifies that USS-level changeover with a target USS (USS-B) is necessary, then

- USS-A informs USS-B about the changeover and sends information about the UAV. The interface between USS-A and USS-B is out of 3GPP scope.

- USS-A informs the UAV and sends information about USS-B e.g address of USS-B and security credentials.

- The UAV and USS-B uses the information provided by USS-A for authentication and authorization purpose.

- USS-B informs the network about the authentication and authorization result.

#### 6.5.2.3	Enhancement for authorization of target USS to use exposure services for the UAV

When the serving USS (USS-A) that has performed the UUAA procedure with the UAV identifies that USS-level changeover with a target USS (USS-B) is necessary, then

- USS-A informs USS-B about the changeover and sends information about the UAV. The interface between USS-A and USS-B is out of 3GPP scope.

- USS-A informs the UAS NF and sends information about USS-B and UAV.

- The UAS NF stores the information that USS-B is authorized to consume the exposure services for the UAV. Note that from the CN side there is no increase in the threat level because USS-B will not be able to access resources which are not accessible by the USS-A, i.e., there will be no additional resources accessible by USS-B but not by USS-A.

- When the UAS NF receives a request from USS-B about UAV, the UAS NF authorizes USS-B by using the stored information and checks whether the location of the UE is inside the geographical area served by USS-B.

### 6.6.3	Evaluation

TBD

*****End of changes*****