3GPP TSG-SA3 Meeting #118	S3-244178

# Hyderabad, India, 14 - 18 october 2024

# Source:	Apple

# Title:	Update solution#4 - MAC CE based solution for LTM security

# Document for:	Approval

Agenda Item:	4.12

# 1	Decision/action requested

Approve the pCR to the living CR of LTM security

# 2	Reference

[1] S3-243689 - Living document on NR mobility enhancement.

# 3	Rationale

This contribution proposes the changes on solution#4, which is based on MAC CE to deliver NCC.

Regarding the 2 ENs below:

Editor’s Note: Whether there is a need to use NCC list is FFS.

Editor’s Note: Whether there is a need to add confidentiality protection on NCC in MAC CE is FFS.

Using NCC list and corresponding index is to conceal the real NCC value, which is only the input of the KgNB calculation, therefore the confidentiality of NCC is not a mandatory requirement. To address those 2 ENs, modification on the solution was proposed.

Regarding the EN below:

Editor’s Note: Whether there is a need to add integity protection on NCC in MAC CE is FFS.

NCC integrity protection is used to detect the proactive attack (e.g. tampering) on the MAC CE, so that UE could verify if the MAC CE has been tampered or not before proceed. Therefore, the integrity protection should not be limited to the NCC, but on the whole MAC CE. It depends on SA3 decision whether to endorse the MAC CE integrity protection in R19 considering the impact it brings to the 5G system. It is proposed to change this EN to a NOTE as:

NOTE: Integrity protection on MAC CE is not in the scope of this solution and should be considered from the perspective of the whole WID.

The evaluaton of the solution is also added.

# 4	Detailed proposal

*** Start of 1st Change ***

## 3.4	Solution #4: MAC CE based solution to deliver the NCC(s)

### 3.4.1	Introduction

This solution proposes the procedure for LTM key derivation. The legacy handover procedure in TS 33.501 clause 6.9.2 will be reused as much as possible. RAN2 procedures on LTM (no DC) are used as the basis.

### 3.4.2	Details

Pre-assumption:

- In this solution, gNB1, gNB2 and gNB3 are the candidate cells, UE is moving from gNB1 to gNB2, then from gNB2 to gNB3.

- Source gNB1 configures the UE, gNB2 and gNB3 with NCC1 to be used for the key derivation.

Figure 3.4.2-1: MAC CE based LTM procedure

Step 0. Source gNB1 configures the UE, gNB2 and gNB3 with NCC1 to be used for the key derivation.

NOTE: Integrity protection on MAC CE is not in the scope of this solution and should be considered from the perspective of the whole WID.

Step 1. gNB1 sends the MAC CE to the UE including NCC1(assuming NCC1 is the next NCC to be used for next handover.

Step2. gNB1 derives K_gNB21 based on NCC1 and sends it to gNB2, the key derivation method follows TS 33.501 Annex A.11.

Step3. gNB1 derives K_gNB31 based on NCC1 and sends it to gNB3, the key derivation method follows TS 33.501 Annex A.11.

NOTE: the order of step 2/3 and step1 follows RAN2 procedure.

Step4. UE triggers the inter-CU switch, handover from gNB1 to gNB2. UE derives K_gNB21 using NCC1.

Step5. After the handover, gNB2 sends NGAP PATH SWITCH COMPLETE message to AMF.

Step6. AMF sends the NGAP PATH SWITCH ACK message back to gNB2, including a new pair {NH2, NCC2}.

Step7. gNB2 derives K_gNB12 based on NCC2 and sends it to gNB1, the key derivation method follows TS 33.501 Annex A.11.

Step8. gNB2 derives K_gNB32 based on NCC2 and sends it to gNB3, the key derivation method follows TS 33.501 Annex A.11.

Step9. gNB2 sends the MAC CE to the UE including NCC2 to be used for next handover.

NOTE: the order of step 7/8 and step9 follows RAN2 procedure.

Step10. UE triggers the inter-CU switch, handover from gNB2 to gNB3. UE derives K_gNB32 using NCC2.

Step11. After the handover, gNB3 sends NGAP PATH SWITCH COMPLETE message to AMF.

Step12. AMF sends the NGAP PATH SWITCH ACK message back to gNB3, including a new pair {NH3, NCC3}.

Editor’s Note: AS security context sync-up after path switch is ffs

### 3.4.3	Evaluation

This solution leverages MAC CE to deliver the NCC, which is aligned with RAN2 motivation for LTM feature. The NCC configuration can be carried within LTM Cell Switch Command MAC CE (TS 38.321) without any further impact on the system.

MAC CE has no integrity protection so far, this is a security concern not only to this solution#4, but also to other solutions based on MAC CE.

*** End of 2nd Change ***