3GPP TSG-SA3 Meeting #118	S3-244207

Hyderabad, India  14 – 18 October 2024

# Source:	Ericsson

# Title:	AIoT: Security threats for KI#4 on protection of information during AIoT service communication

# Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

This document proposes to add security threats for the KI on protection of information during AIoT service communication.

# 2	References

[1]	3GPP TR 23.700-13

[2]	3GPP TR 33.713

# 3	Rationale

This contribution proposes to add security threats to the KI on protection of information during AIoT service communication.

# 4	Detailed proposal

**** Start of Change ****

## 5.4	Key issue #4: Protection of information transmitted during AIoT service communication

### 5.4.1	Key issue details

As per TS 22.369 [2], Ambient power-enabled IoT (AIoT) services aim to support various use cases, including inventory taking, sensor data collection, asset tracking, and actuator control. These services intended to operate with lower power consumption and complexity than the existing IoT technologies such as eMTC, NB-IoT, and RedCap. To fulfil these requirements, AIoT devices require a communication capability.

From a security perspective, security mechanisms to protect the information transmitted during AIoT service communication need to be supported. Failure to provide such security mechanisms will lead to various attacks such as eavesdropping, manipulation and/or unauthorized transmission of the information during AIoT service communication.

Therefore, this key issue focuses on how to protect the information transmitted during AIoT service communication considering the specific use cases that are differentiated from the existing IoT technologies.

### 5.4.2	Security threats

An attacker can acquire data transmitted to/from AIoT devices by eavesdropping messages if the communication of AIoT service is not confidentiality protected.

An attacker can manipulate information during communication of AIoT service if the communication of AIoT service is not integrity protected.

An attacker can replay a message if replay protection is not activated.

### 5.4.3	Potential security requirements

TBD.

**** End of Change ****