3GPP TSG-SA3 Meeting #118	S3-244208

Hyderabad, India  14 – 18 October 2024

# Source:	Ericsson

# Title:	Updating KI#5 Authentication in Ambient IoT service

# Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

This document proposes to update KI#5 Authentication in Ambient IoT service.

# 2	References

None

# 3	Rationale

The EN on identifying further threats needs to be addressed. New threats to the existing threats are added. A new security requirement is proposed on the credentials for authentication at the application layer when application layer authentication is required.

# 4	Detailed proposal

**** START OF CHANGE ****

## 5.5	Key Issue #5: Authentication in Ambient IoT service

### 5.5.1	Key issue details

The TR 23.700-13 [4] studies the architecture support of Ambient Internet of Things (AIoT) device, considering the service requirements for ambient power-enabled IoT device. In TR 23.700-13 [4], the validation of the AIoT device identity and authentication are explicitly mentioned.

### 5.5.2	Threats

In the air interface, an attacker may impersonate the victim device and report fake identification to the network side. If the billing is based on per AIoT device’s identity, the fake identity may lead to charging problem. This can be used by an adversary to steal an AIoT device by replacing the AIoT device with a fake device, which can cause a great loss to the owner of the device. Whether fake identities pose a threat in case of inventory use case, is up to the individual deployments risk profile.

A device can obtain unauthorized access to network resources. A malicious device can send malicious traffic towards the mobile network e.g., as part of a DDoS attack.

Over the air interface, an adversary can impersonate a legitimate network and send command — e.g., disablement command will permanently deactivate a device.

### 5.5.3	Potential security requirements

The 5G system shall provide a means to perform mutual authentication between the AIoT device and the network.

NOTE: If solution reuses the existing authentication framework (e.g., 5G-AKA, EAP-AKA’, other EAP methods for SNPN), the existing UE security requirements in TS 33.501[5] apply. Otherwise, the core network entities, if any, that are used for authentication are dedicated for Ambient IoT service.

Editor’s Note: Secure storage and processing of credentials (on the AIoT device and network side) for solution not reusing existing authentication framework as per the note above is ffs.

Editor’s Note: Whether authentication of AIoT device is mandatory is ffs.

Editor’s Note: Whether one-way authentication needs to be defined is ffs.

Editor’s Note: Whether authentication of AIoT device can be done on application layer is ffs.

If authentication is required at the application layer, the mutual authentication between the AIoT devices and the 5G network shall be based on connectivity credentials, which may or may not be the same as the credentials required for authentication on the application layer.

**** END OF CHANGE ****