3GPP TSG-SA3 Meeting #118	S3-244253

Hyderabad, India 14 - 18 October 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 2052 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Signaling Monitoring Requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone, Nokia, Nokia Shanghai Bell, Ericsson, Telefonica |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | MonStra-Sec |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 5G SBA NF service based interfaces communicate using encrypted signaling. This poses a problem for traditional probing solutions when it comes to capturing the desired data without breaching security. There is no standardized mechanism in 5GC to securely provide raw protocol signaling data to external network monitoring systems. Operators require to maintain operational observability to troubleshoot faults and run health checks. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce security requirements and solution to secure interactions between 5GC and external network monitoring systems. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Signalling monitoring for observability of 5G SBI signalling cannot be run securely leading to proprietary implementations that increase integration costs and efforts in a multivendor 5GC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 4.2.0, 4.2.x (new), 5.9.1, 5.9.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start of 1st Change ***

## 3.1	Definitions

Secondary node: As defined in TS 37.340 [51].

Signalling Monitor Consumer: It refers to the external monitoring system that receives the copies of the signalling traffic sent or received by a network function (NF)

Signalling Monitor Producer: It refers to the 5GC NF or to an entity on behalf of an NF that sends copies of signalling traffic exchanged with other network functions.

Editor's Note: These terms are to be defined in SA5, so these definitions can be revised accordingly.

subscription credential(s): The set of values in the USIM and in the home operator's network, consisting of at least the long-term key(s) and the subscription identifier SUPI, used to uniquely identify a subscription and to mutually authenticate the UE and 5G core network.

*** Start of 2nd Change ***

## 4.2	Security at the perimeter of the 5G Core network

### 4.2.0	General

The security specified in this clause applies to both roaming, PLMN interconnect and monitoring scenarios in which 5G Core NF(s), or an entity on their behalf, interact with external monitoring systems for troubleshooting or surveillance purposes.

NOTE: External monitoring systems are fully under the control of the MNO but in different trust zones (see 5.9.1)

*** Start of 3rd Change ***

### 4.2.X	Signalling Monitoring security

The Signalling Monitoring security introduces functionality to enforce security mechanisms applied by network functions, or an entity on their behalf, that are requested to securely send a copy of 5GC NF signalling traffic to an external monitoring system.

Security functionality is also introduced at configuring the signalling monitoring.

*** Start of 4th Change ***

### 5.9.1	Trust boundaries

It is assumed for the set of requirements in this sub-clause that mobile network operators subdivide their networks into trust zones. Subnetworks of different operators are assumed to lie in different trust zones. Messages that traverse trust boundaries shall follow the requirements in sub-clause 5.9.2 to 5.9.x of the present document, if not protected end to end by NDS/IP as specified in TS 33.210 [3].

*** Start of 5th Change ***

### 5.9.X	Requirements for Monitoring 5GC Signaling Traffic

#### 5.9.X.1 Security requirements for the configuration of signalling monitoring

The solution shall support mutual authentication between the Signalling Monitor Producer at the 5GC (for configuration/activation of the functionality) and the Signalling Monitor Configuration Consumer.

Editor's Note: The terms used in the new Signalling monitoring functionality are to be defined in SA5, so the terminology used in this clause can be revised accordingly.

NOTE 1: The Signalling Monitor Consumer can be part of the MNO management or reside in the monitoring system domain.

The solution shall support authorization to the Signalling Monitor Consumer for configuration of signaling monitoring.

NOTE 2: Local policy-based authorization should be supported by the Signalling Monitor Producer, according to 3GPP TS 28.533.

The solution shall support integrity protection, replay protection and confidentiality protection for communication between the Signalling Monitor Producer at the 5GC and the Signalling Monitor Consumer for configuration of signaling monitoring.

#### 5.9.X.2 Security requirements for the transfer of signalling monitoring data

The solution shall support mutual authentication between the Signalling Monitor Producer at the 5GC and Signalling Monitor Consumer.

NOTE:  The Signalling Monitor Consumer is outside the scope of this specification.

The solution shall provide confidentiality protection of the interface between the Signalling Monitor Producer at the 5GC and the Signalling Monitor Consumer.

The solution shall provide integrity protection of the interface between the Signalling Monitor Producer at the 5GC and the Signalling Monitor Consumer

The solution shall provide replay protection of the interface between the Signalling Monitor Producer at the 5GC to the Signalling Monitor Consumer.

*** End of Changes ***