3GPP TSG-SA3 Meeting #118	S3-244278

Hyderabad, India 14 - 18 October 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	Solution for KI#2 on Privacy of user sensitive information

Document for:	Approval

Agenda Item:	5.18

# 1	Decision/action requested

Approve the solution added to TR 33.721

# 2	References

[1]	3GPP TR 23.700-21 Study on Application enablement architecture for mobile metaverse services

[2]	3GPP TR 33.721 Study on security aspects of 5G Mobile Metaverse services

[3]	3GPP TS 33.501 Security architecture and procedures for 5G system

# 3	Rationale

According to 4.2	Key Issue #2: Exposure of user sensitive information of TR 23.700-21, ensuring appropriate user consent has been obtained is a critical aspect when handling sensitive information relating to or collected from a user, their devices or the applications installed at their devices. For instance, with the expected capability to access, manage and expose user specific avatar related information through the enabler layer it is of utmost importance to capture the consent of the user. Another example are, digital assets, where ownership rights are considered to be classified as sensitive information and the owner's consent must be obtained before other users or third party(service provider) can use them. Accordingly, the KI#2 was created in TR 33.721, which requires the 5G system to provide a means for privacy protection of user sensitive information during exposure of user specific information (e.g. user identity, user location) in localized mobile metaverse services through the application enabler layer.

Solution#4 in the present TR proposed to get authorization from resource owner or check user consent in UDM when provide metaverse service to a metaverse service consumer. As spatial anchor, spatial map discovery are supported in solution#1 (clause 7.1) and solution#8 (clause 7.8) of TR 23.700-21, this contribution proposes a solution to check user consent already in discovery procedure, and return list of spatial anchors, spatial maps which matches user consent to the metaverse service consumer. In addition, the solution addresses privacy concerns when expose avatar profile to a application server.

# 4	Detailed proposals

All content is new.

# * * * * First change* * * *

## 6.x	Solution #x: Privacy protection during metaverse service discovery

### 6.x.1	Introduction

This solution addresses Key Issue #2 on privacy of user sensitive information.

According to 4.2	Key Issue #2: Exposure of user sensitive information of TR 23.700-21, ensuring appropriate user consent has been obtained is a critical aspect when handling sensitive information relating to or collected from a user, their devices or the applications installed at their devices. For instance, with the expected capability to access, manage and expose user specific avatar related information through the enabler layer it is of utmost importance to capture the consent of the user. Another example are, digital assets, where ownership rights are considered to be classified as sensitive information and the owner's consent must be obtained before other users or third party(service provider) can use them.

Spatial anchor, spatial map discovery are supported in solution#1 (clause 7.1) and solution#8 (clause 7.8)  of TR 23.700-21 [2], this contribution proposes a solution to check user consent in discovery procedure, and return list of spatial anchors, spatial maps, which match user consent, to the metaverse service consumer or SEAL client. In addition, the solution addresses privacy concerns when expose avatar profile to a VAL server.

### 6.x.2	Solution details

Figure 6.x.2-1 Procedure of privacy protection during metaverse service discovery

Precondition:

A list of spatial anchors, spatial maps and avatars are created, personal data required to support each spatial anchor, spatial map or avatar is registered.

1. UE initiates a discovery request to SEAL server to get the list of spatial anchors or spatial maps, application specific UE Id is included in the request.

2. SEAL server requests the UDM to get the user consent for metaverse services based on the UE Id.

3. UDM sends the user consent for the metaverse services to the SEAL server.

4. SEAL server processes the user consent along with the personal data required to support registered spatial anchors or spatial maps.

5. SEAL server sends a list of spatial anchors or spatial maps which personal data requirements match UE consent.

6. UE sends a request to VAL server to access metaverse service along with an Avatar ID and application specific UE Id.

7. VAL server requests for relevant avatar (i.e. avatar profile) from SEAL server based on Avatar ID and the UE Id.

8. SEAL server requests UDM for the user privacy requirements based on the UE Id

9. Based upon user consent and privacy requirements, SEAL server generates avatar profile only containing relevant fields and protecting personal sensitive information.

10. SEAL server sends avatar profile respecting user consent and privacy requirements to the VAL server.

### 6.x.3	Evaluation

TBA.

# * * * * End of changes * * * *