3GPP TSG-SA3 Meeting #118	S3-244283

Hyderabad, India  14 - 18 October 2024

# Source:	Nokia, Nokia Shanghai Bell

# Title:	Evaluation summary of solutions for KI#2.1

# Document for:	Approval

Agenda Item:	5.14

# 1	Decision/action requested

It is required to approved this pCR to be part of clause 7 in TR 33.749

# 2	References

[1]	3GPP TR 33.749 v0.3.0 “Study on security aspects of enhancement of support for edge computing in the 5G Core (5GC) phase 3”

# 3	Rationale

The contribution is intended to make an evaluation summary of the solutions proposed in [1] to address the threats and requirements of the KI#2.1. The proposed table summarizes whether the threats and requirements have been addressed, and in case they have, it indicates the main principles. It is also indicates the IE used for verification, and finally some considerations in terms of impact in the actual 5GS.

# 4	Detailed proposal

*********************START of CHANGES********************

# 7	Conclusions

Editor’s Note: This clause contains the agreed conclusions that will form the basis for any normative work.

## 7.X	Evaluation summary of the solutions for KI#2.1

The following table 7.2-1 shows concisely how the threats and security requirements of KI#2.1 have been addressed by the different solutions in clause 6 of the present document.

|  | Threat #1¦ Req. #1 (clauses 5.3.1.2.1 & 5.3.1.3.1) | Threat #2 ¦ Req. #2 (clauses 5.3.1.2.2 & 5.3.1.3.2) | IE used for verification | Impact / Observation |
| --- | --- | --- | --- | --- |
| Solution #1 (Clause 6.1) | User information provided by the EEC is private IP address The solution verifies the provided IP address (private) by querying the UPF the NAT mapping table. | It is not addressed. It uses both private and public IP address of the UE. | Private IP | It requires NAT functionality to be embedded in UPF. It requires the UE to include private IP in the traffic at application level. It leverages the mapping between public and private IP to verify the provided (private) IP. It exposes the private and public IP addresses of EEC to EES (EES can be inside or outside the operator domain). |
| Solution #2 (Clause 6.2) | User information provided by the EEC is a hash value computed by using private information (e.g., SUPI) + counter. Nnef_UEId API is invoked with hash value and UE IP address. The hash is used to verify the IP address provided by AF to NEF in the API invocation. | It is not addressed if it uses private IP address of the UE to invoke 5GC API, that can be used to break the user privacy. | Hash value | AF needs to be able to reach UE (AF initiates the communication in step 1) to trigger the hash generation. The data used for generating the hash determines the strength of the verification. The hash can be subject to brute-force type of attacks. It prevents replay attacks from malicious AF, since the hash is different in every API invocation. It's unclear which IP address, private or public IP address, is used in Nnef_UEId API. |
| Solution #3 (Clause 6.3) | User information provided by the EEC is a temporary ID, generated and provided by the 5GC for PDU session. The 5GC verifies the temporary ID provided by the AF with the one generated for the UE. | The user information provided by the EEC is a temporary ID assigned by the Core, thus the privacy of the UE is preserved. | Temporary ID | The UE needs to request the temporary ID in the PDU session establishment/modification request and provide it to the EEC. It does not require to expose any network information outside the 5GC, such as to EDGE servers. |
| Solution #4 (Clause 6.4) | User information provided by the EEC is A-KID coming from AKMA procedure. AKMA guarantees that the information provided is trusted. | 5G UE Id is not required to be fetched as per authentication service provided by AKMA. The privacy of the UE is implicitly preserved. | A-KID/GPSI | The solution requires AKMA to be implemented from the operator It does not require to expose any network information outside the 5GC to EDGE servers. Existing Kaf can be reused to protect sensitive information, e.g. location, sent from EAS to UE. |
| Solution #5 (Clause 6.5) | EEC provides a signed version of the required data, i.e., private IP address, along with a certificate to prove their authenticity. | It is not addressed. Signature only provides integrity protection but does not provide confidentiality of the information, therefore the EES can still collect the IP address | Certificate /Signature | It requires an additional CA system for the EEC (UE) trusted by all EES, or other schemas like cross-certification are to be considered. It requires that the certificate includes also the IP address of UE, which could change dynamically. |
| Solution #6 (Clause 6.6) | The EEC provides a token-based solution that directly allows EES to retrieve the corresponding 5G UE ID. UE ID server ensures the authenticity of the token. | The user information provided by the EEC is a Token that does not contain privacy concerning information, thus the privacy of the UE is preserved. | UE ID token | The solution introduces a new Network Function (UE ID Server) It does not require to expose any network information outside the 5GC to EDGE servers. |
| Solution #7 (Clause 6.7) | The EEC provides the PDU session ID along with the IP address. The NEF uses the IP address to retrieve the PDU session ID from UDM and compare it with the one received from EES. | It is not addressed. The private IP address is still provided along with the PDU session ID. | PDU Session ID | The UE privacy protection relies on the usage of the PDU session ID. Therefore, it is required to study, whether the PDU session ID is a sensible information and whether brute-force attacks are possible on it. |
| Solution #8 (Clause 6.8) | The EEC provides IP address and access token (OAuth), which includes UE ID Access token is used for the verification of the IP address. | It is not addressed The private IP address is sent along with the access token | Access token | Comment: ENs need to be resolved. |