3GPP TSG-SA3 Meeting #118	S3-244286

Hyderabad, India  14 – 18 October 2024

# Source:	Nokia, Nokia Shanghai Bell

# Title:	Fixing Ens in solution #4

# Document for:	Approval

Agenda Item:	5.14

# 1	Decision/action requested

It is required to approve this contribution intended to fix the Editor’s notes.

# 2	References

[1]	3GPP TR 33.749 v0.3.0 “Study on security aspects of enhancement of support for edge computing in the 5G Core (5GC) phase 3”

# 3	Rationale

This contribution is intended to clarify the points arose by the following editor’s note, and accordingly proposes to remove it.

Editor’s Note: The evaluation should state the applicability of the solution, i.e. what is the solution solving and what are the assumptions.

It is clarified that the solution assumes that AKMA is implemented in the network, so there is nothing additional coming from the EEC to be verified (AKMA already provides the authentication service). Therefore, the solution, AKMA feature itself, implicitly addresses the first and second requirements (clauses 5.3.1.3.1 and 5.3.1.3.2) simply by reusing the A-KID.

# 4	Detailed proposal

************************ START of CHANGES****************************************

### 6.4.3	Evaluation

The main impact and assumption of the solution is that it requires the support of AKMA feature in the 5G system. AKMA feature itself, implicitly addresses the first and second requirements (clauses 5.3.1.3.1 and 5.3.1.3.2) simply by reusing the A-KID

The 5G UE Id is not required to be fetched by the AF, since 5G Core through AKMA feature provides already the authentication service, and the trust between UE, 5G Core and AF. Therefore, the privacy of the UE is implicitly preserved.

The solution avoids the need to retrieve a 5G system UE Id to be used in the invocations of the NEF APIs. AKMA itself ensures the reliability of A-KID, that it is being reused in the AF invocation towards the NEF, instead of using any other identifier provided by the EEC to be further verified in the procedure.

************************ END of CHANGES****************************************