3GPP TSG-SA3 Meeting #118	S3-244327

Hyderabad, India, 14th – 18th October 2024				merger of S3-243845, S3-243931, S3-243873, S3-243999, S3‑244191

# Source:	MITRE-FFRDC, Lenovo, Johns Hopkins University, BT, Ericsson, AT&T, TMobile-US

# Title:	eZTS KI 1 Conclusions

# Document for:	Approval

Agenda Item:	5.1

# 1	Decision/action requested

This pCR proposes conclusions for TR 33.794[1] to specify requirements for security monitoring and evaluation.

# 2	References

[1]	3GPP TR 33.794 Study on enablers for Zero Trust Security

[2]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

# 3	Rationale

The present document studies the use cases for security evaluation and monitoring in clause 5.1. This study identified and evaluated the relevant data to be collected to aid in security evaluation and monitoring for each use case.

This document proposes 5G system-level security requirements to address KI#1 Data exposure for security evaluation and monitoring. The requirements also address the use cases for security evaluation and monitoring described in clause 5.1.

In tdoc S3-243839 ‘LS on Way forward on working agreement #63’, it says “Additionally, SA3 is requested to define requirements in TR 33.794 to identify data to be collected that may be relevant to enable security evaluation and monitoring. The conclusion from the TR 33.794 forms the basis of further work to enable ZTS concepts as relevant to an operator network.”. This document proposes requirements to address this request from SA#105.

# 4	Detailed proposal

SA3 is kindly requested to approve the below change to TR 33.794 [1]

********************** Start of 1st Change ****************************

# 8	Conclusions

Editor’s Note: This clause contains the agreed conclusions that will form the basis for any normative work.

## 8.1	Key Issue #1: Data exposure for security evaluation and monitoring

The security incidents or scenarios in SBA where data can be collected in the SBA layer includes

1)	authentication and authorization failure event;

2)	unexpected setup of TLS session and API invocation related to unauthorized reconnaissance;

3)	malformed message event;

4)	high service load;

5)	unexpected SBI call flows; and

6) unexpected use of APIs exposed by services in SBA layer

The key issue will be addressed by requirements for data collection to enable security evaluation and monitoring on the SBA layer. No new interface nor protocol will be specified as result of the work in this report.

The following requirements address KI#1 Data exposure for security evaluation and monitoring. The requirements also address the use cases for security evaluation and monitoring described in clause 5.1.

General requirements for security event logs:

1.	The NF supports the generation of security event logs.

Security event logs contain security relevant events data. Requirements related to security relevant events data for monitoring:

a.	The NF collects information on the SBA layer about received malformed messages that deviate from the 3GPP specified messages or are considered invalid according to the protocol specification and network state are to be logged. (Clause 5.1.1)

b.	The NF collects information about events involving a massive number of incoming messages on the SBA layer. (Clause 5.1.2).

c.	The NF collects information about authentication and authorization failure on the SBA layer. (Clause 5.1.3).

d.	The NF collects information about potential replay attacks on the SBA layer. (Clause 5.1.6).

Editors note:	Bullet d) is FFS.

Editors note:	Additional details if any are FFS.

*** End all CHANGES ***