3GPP TSG-SA3 Meeting #118	S3-244377

# Hyderabad, India, 14th - 18th October 2024            Merger of S3-244143, S3-244241, S3-244221, S3-243971 and S3-243970

# Source:	Qualcomm Incorporated

# Title:	Conclusion of KI#2

# Document for:	Approval

Agenda Item:	5.12

# 1	Decision/action requested

This contribution proposes a conclusion of KI #2.

# 2	References

[1]	TR 33.743

# 3	Rationale

This contribution proposes to add a conclusion of Key Issue #2.

Particularly, for IP PDU type of muti-hop UE-to-UE relay, it is proposed to use the security procedures in solution #12, #14 and #15 as a basis for normative work.

# 4	Detailed proposal

It is proposed that SA3 approve the below pCR for inclusion in the TR [1].

***** START OF CHANGES *****

## 7.x	Key Issue #2: Security for Multi-hop UE-to-UE Relay

The following statements are agreed as a basis for normative work:

For 5G ProSe Multi-hop UE-to-UE Relay discovery of IP PDU type:

- The security procedures for both relay discovery among 5G ProSe UE-to-UE Relays and relay discovery between an 5G ProSe End UE and 5G ProSe UE-to-UE Relay are based on solution #14.

For 5G ProSe Multi-hop UE-to-UE Relay communication traffic of IP PDU type:

- The security procedure for both PC5 link establishment among 5G ProSe UE-to-UE Relays and PC5 link establishment between an 5G ProSe End UE and 5G ProSe UE-to-UE Relay is based on solution #12 and solution #15.

For 5G ProSe Multi-hop UE-to-UE Relay discovery of non-IP PDU type (Ethernet or Unstructured):

- For discovery with Model B, the 5G ProSe End UE and 5G ProSe Multi-hop UE-to-UE Relay are provisioned with the discovery security materials associated with an RSC from the 5G PKMF/5G DDNMF in their own HPLMN, reusing the procedures specified in clause 6.1.3.2 of TS 33.503 [5].

- For discovery with Model B, the 5G ProSe End UE and 5G ProSe Multi-hop UE-to-UE Relay use the mechanisms as specified in clause 6.1.3.3 of TS 33.503 [5] as baseline for the normative work to protect the discovery messages.

For 5G ProSe Multi-hop UE-to-UE Relay communication traffic of non-IP PDU type (Ethernet or Unstructured):

- The security procedure specified in clause 6.2 is used as a baseline to establish a secure PC5 link for each hop among 5G ProSe End UDs and 5G ProSe UE-to-UE Relays.

- Both UP-based and CP-based security procedures specified in clauses 6.3.3.2 and 6.3.3.3 of TS 33.503 [5] can be used as a baseline for a secure PC5 link establishment for each hop among 5G ProSe End UEs and 5G ProSe UE-to-UE Relays if 5G ProSe UE-to-UE Relays are in-coverage.

Editor’s Note: Further conclusion on 5G ProSe Multi-hop UE-to-UE Relay of non-IP PDU type is FFS.

***** END OF CHANGES *****