3GPP TSG-SA3 Meeting #118	S3-244485

Hyderabad, India  14 – 18 October 2024

# Source:	Ericsson

# Title:	New Solution to KI#3 Privacy by protecting AIoT device identifiers

# Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

This document proposes a new solution to KI#3 Privacy by protecting AIoT device identifiers.

# 2	References

None

# 3	Rationale

Public key encryption is too expensive for AIoT devices. Therefore, AIoT devices cannot compute SUCIs as specified in 5G. This is a problem to solve to fulfill the potential security requirement described in KI#3. One solution is that the network computes SUCI on behalf of the AIoT device and securely transports the SUCI to the AIoT device that the AIoT device uses in the following inventory procedure.

# 4	Detailed proposal

**** START OF CHANGE ****

## 6.X	Solution #X: Providing a network-computed AIoT concealed device identifier (AICI) to an AIoT device

### 6.X.1	Introduction

This solution addresses KI#3: Privacy by protecting AIoT device identifiers.

### 6.X.2	Solution details

The solution proposes a method for the 5G network to compute a AIoT Concealed Device Identifier (AICI) and provide the SUCI to the AIoT device in a command message. Once an AIoT device is identified, e.g., after a successful completion of inventory procedure, the network can send a command message on the downlink channel to the AIoT device. In the downlink command message, the network includes an AICI, which is computed based on the long-term identifier of the AIoT device using the public key of the network. The downlink command message is both confidentiality and integrity protected using keys derived from a shared key between the network and the AIoT device. Figure 6.3.2-1 presents a high-level message flow of the solution.

Figure 6.X.2-1: Procedure for delivering a AICI to an AIoT Device

The figure is self-explanatory, therefore, the steps are not explained step-by-step. Exact content of the messages exchanged, and details about authentication challenge, computing response to the challenge, and deriving the keys Kenc and Kint are not described because these details have to be adjusted with the authentication protocol that is finally agreed.

NOTE: If the AIoT device does not have a network-computed AICI, for example, in the very first time of the device’s life cycle, then the AIoT device computes AICI using null scheme. This happens only in the beginning. To avoid using null scheme in the first time, a network can choose to provision every AIoT device with a network-computed AICI before they are handed out to their users.

Editor’s Note: The content of the paging message and how the device decides to respond to the paging message based on the content of the paging message is FFS.

Editor’s Note: Synchronization of AICI is FFS

Editor’s Note: Whether AICI is to be computed and decrypted in SIDF/UDM is FFS

Editor’s Note: Whether AIoT device can update and store AICI is FFS

### 6.X.3	Evaluation

TBD

**** END OF CHANGE ****