# 3GPP TSG-SA3 Meeting #118 	S3-243855

# October 14 –18, 2024  Hyderabad, IN

# Source: 	Interdigital

# Title: 	Solution for Lightweight AIoT ID privacy

# Document for: 	Approval

Agenda Item: 	5.9

# 1	Decision/action requested

Approve the pCR to TR 33.713

# 2	References

[1]	TS 33.501 Security architecture and procedures for 5G System

[2]	TR 38.769 Study on solutions for Ambient IoT (Internet of Things) in NR

[3]	TS 22.369 Service requirements for Ambient power-enabled IoT

[4]	RAN2#125bis chairman notes https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_125bis/Inbox/Chair_Notes/R2_125bis_ChairNotes_24-04-19_final.docx

# 3	Rationale

This contribution proposes a lightweight ID privacy solution. It was submitted to SA3#116 but not treated and was reviewed and received comments at the SA3 offline AIOT Security call #1 on July 8, 2024. Comments received during the call are reflected in the proposed solution.

# 4	Detailed proposal

SA3 is asked to approve the following pCR.

*** Start of 1st Change ***

6.X			Solution #X: Lightweight AIOT ID privacy based on hashes

### 6.X.1			Introduction

The assumption of this solution is AIoT device can not support 5G-AKA due to power or computational resource limitation.

The existing Key issue #3, Privacy by protecting AIoT device identifiers, specifies the following requirement:

Mechanisms for mitigating privacy threats (described above) by identifying, linking, and tracking the identifiers of AIoT Device(s) shall be supported.

In addition, RAN2#125bis chairman notes [4] detail the following baseline procedure:

…

3. RAN2 will support two use cases, “inventory” and “command”.  The definition, detailed wording is FFS

4. Baseline:

Step A: Based on the service request, the reader sends the Initial Trigger Message indicating device(s) that need to respond; Details FFS

Step B: Triggered device(s) performs the random access-like procedure, if needed; Details FFS

Step C: The device may perform the data communication with the reader as needed,: Details FFS

…

This solution aims to address both, the KI#3 of the present document and the baseline procedure described in the RAN2#125bis agreement.

Moreover, the proposed procedure for obfuscating the AIoT Device AIOT_ID describes the “inventory” use case but is equally applicable to the “command” use case.

### 6.X.2			Details

The simplified call flow associated with the proposed solution is presented below.

Figure 6.X.2-1: Call flow for Lightweight AIOT ID privacy based on hashes

Steps associated with the call flow in Figure 6.X.2-1:

0. Configuration and provisioning step

0a. AIOT Device is configured/provisioned with AIOT_ID

0b. RAN Reader/Intermediate Node is configured/provisioned with a list of AIOT_IDs

1.	AF sends an Inventory Operation Request with the following information: target area for the operation, client which requests the operation, and match information used to filter and discover the target AIoT devices for the operation.

2.	The NEF authorizes the AF request. If the AF request is authorized, The NEF discovers the AIoT function using the information in the Inventory Operation Request, e.g. using the target area for the operation to discover the AIoT function from NRF. If the target area for the operation matches the AIoT service area of the AIoT Function, the NRF returns the information for the AIoT function to the NEF.

3.	The NEF forwards the Inventory Operation information to each of the selected AIoT functions.

4.	The AIoT function discovers and selects a reader or an Intermediate Node to perform Inventory Operation according to the Inventory Operation information. e.g. using the target area for the operation to discover the readers. If the target area for the operation matches the AIoT service area supported by the readers, those readers can be selected to execute the inventory operation.

5.	The AIoT Function sends an Inventory Request with the Inventory Operation information for each selected reader.

6.	The selected reader executes the inventory operation towards the target AIoT Devices.

Note that step 6 may be eavesdropped, manufactured, and replayed by an attacker. An appropriate authorisation of the Reader and freshness control to prevent message 6 replay will be needed to remedy such attacks.

7. The AIoT Device performs AIOT_ID selection based on matching of hashed AIOT_ID

a. The AIoT Device computes a hash using RAND_READ as salt for each AIOT_ID

b. The AIoT Device compares the hash values of its provisioned AIOT_ID and AIOT_ID hashes from the broadcast message to find at least one matching hashed AIOT_ID.

c. The AIoT Device selects an AIOT_ID that has a matching hash.

Note that step 7 may create an overhead for the AIOT that may lead to a resources depletion attack on AIoT Device. This overhead is either lighter or comparable with other methods for confidentiality protection of the AIoT Device identity.

8. The AIoT Device sends a Registration Request containing hashed obfuscated AIOT-ID

9. The RAN Reader or Intermediate Node performs AIOT_ID selection based on matching of hashed AIOT_ID

a. compute a hash using RAND_READ for each AIOT_ID

b. check that the hashed AIOT_ID received from the AIOT Device matches one of the stored AIOT-ID from step 0b

10. The RAN Reader or Intermediate Node sends the Inventory Report with AIOT_ID to AMF/AIOT AF

11-12..	AIoT Function reports the operation result to the AF vie NEF/AF.

### 6.X.3			Evaluation

This solution addresses the requirement of Key issue #3.

This solution proposes a lightweight privacy protection method for the AIoT Device identity. The proposed method provides lightweight confidentiality protection of AIoT Device identifiers.

Further evaluation is FFS.

*** End of 1st Change ***