3GPP TSG-SA3 Meeting #118	S3-243873

Hyderabad, India, 14th – 19th October 2024

# Source:	Johns Hopkins University APL, British Telecom

# Title:	eZTS Security Event Data Record Format

# Document for:	Approval

Agenda Item:	5.1

# 1	Decision/action requested

This pCR proposes conclusion for TR 33.794 to specify content and format of security event data records.

# 2	References

[18]	3GPP TS 23.501: “System architecture for the 5G System (5GS); Stage 2”

# 3	Rationale

The present document proposes a format for security event data records.

# 4	Detailed proposal

SA3 is kindly requested to approve the below change to TR 33.794.

********************** Start of 1st Change ****************************

# 8	Conclusions

Editor’s Note: This clause contains the agreed conclusions that will form the basis for normative work.

## 8.X Data collection

### 8.X.1 Security event data record

Table 0-1 Security Event Data Record Format indicates the content and format of security event data records. A security event is an indication of a change in status of one or more monitored security attributes listed in clause 8.1. A threshold may be applied to the type and level of change as indicated by an operator-configured policy. A security event log record captures the data associated with the change in status of one or more monitored security attributes listed in clause 8.1.

Refer to 23.501Annex E.1 [18] for methods that may be employed for communicating data records between Network Functions.

| Security Log Info Category | Attribute Name | Data Type | P | Description |
| --- | --- | --- | --- | --- |
| Security event type | SecEventType | string | Required | Identifies the security event type associated with a security data record. Examples of security event types include: admin, execution, protocol, authentication, and authorization. |
| Security event code | SecEventCode | Uinteger | Required | Identifies the security event associated with a security data record. A security event code identifies a monitored security event attribute. Examples of security event codes include: User privlege escalation, Memory access violation, Excessive processing resource consumption, Excessive rate of received messages, Replayed messages, failed authentication procedure, Malformed protocol message, Malformed OAuth Token. |
| Data Producer Entity | NF Instance ID | string | Required | Identifies the NF producing the security data record. The format of the NF Instance ID is described in TS 29.571, clause 5.3.2 Simple Data Types. |
|  | NF Service ID | string | Optional | Identifies the service within an NF producing the security data record. The format of the NF Service ID is described in TS 29.571, clause 5.3.2 Simple Data Types. This is optional because the soure of the security event may be an entity which is not an NF Service. |
|  | NF Profile | string | Required | Identifies attributes of the NF producing the security data record. The NF Profile is described in TS 29.510, clause 6.1.6.2.2. It includes FQDN, IPv4Addresses, IPv6Addresses, nfServiceList, scheme (e.g., http), and port number. |
|  | Event Src_IP_address_port | string | Optional | Identifies the address/port of the entity which originated the security event. This is optional because the source of the security event may be an entity which is not associated with an IP address, such as a hypervisor or a container manager. |
| Data Transmitting entity | IpEndPoint | string | Required | Identifies the NF which is sending the data record. The format of the IpEndPoint is described in TS 29.510 clause 6.1.6.2.5. |
| Security event start time | SecEventStart | string | Required | Identifies the time-of-day at which the security event. The format of the SecEventStart is described in RFC 9957. |
| Security event end time | SecEventEnd | string | Required | Identifies the time-of-day at which the security event ended. The format of the SecEventStart is described in RFC 9957. |
| Security event counter | SecEventCount | Uinteger | Optional | Provides a count of security events that occurred between the Event Start Time and the Event End Time. |
| Security event timestamp clock | ClockSource | string | Required | Identifies the clock source used to mark the security event start and end times. Examples of ClockSource parameter values include: Unknown, Internal free-run, NTP client, IEEE 1588 PTP client. |
|  | ClockStatus | string | Optional | Provides operational status associated with clock source used to mark the security event start and end times. Example operational status values include: Not sysnchronized, Attempting to synchronize, Synchronized. |
| Security data record version | Data Record Version | string | Required | Identifies the version number of the security log record specification. |
|  | Data Record ExtensionID | string | Required | Identifies the data record extension identifier associated with the security data record. |
| Security data record extension information | ExtensionFieldLen | Uinteger | Required | Identifies the length of the security data extension field. |
|  | ExtensionFieldInfo | string | Optional | Provides the security data extension information. The content of this field is not specified. Note that best practices avoid including persistent private credentials (eg, passwords, hashes, or crypto keys) within event data records. |

Table 0-1 Security Event Data Record Format

********************** End of 1st Change ****************************