3GPP TSG-SA3 Meeting #118	     S3-243907

Hyderabad, India 14 – 18 October 2024

Source:	Google

Title:	A server-initiated ACME certificate renewal for 5G SBA

Document for:	Approval

Agenda Item:	5.4

# 1	Decision/action requested

Approval of a new solution to address KI#5 in TR 33.776.

# 2	References

[1]	3GPP TR 33.776: “Study of ACME for Automated Certificate Management in SBA”

[2]	IETF RFC 8555: “Automatic Certificate Management Environment (ACME)”, 2019

[3]	3GPP TS 33.310: “Network Domain Security (NDS); Authentication Framework (AF)”

# 3	Rationale

This contribution proposes a solution that addresses Key Issue #5 - Certificate renewal, as defined in TR 33.776 [1]. The solution is based on use of the ACME protocol, as defined in RFC 8555 [2].

# 4	Detailed proposal

***	BEGINNING OF CHANGES (all text new) ***

## 6.x	Solution #x: A server-initiated ACME certificate renewal

### 6.x.1	Introduction

This solution addresses KI#5

#### 6.x.2.1	Solution overview

A certificate renewal in the context of 5G SBA with ACME may be defined as issuance of a new certificate to replace the old certificate. This solution uses similar call flows as client-initiated ACME-based certificate renewal procedure. However, unlike client-initiated  ACME-based certificate renewal as described in RFC 8555 [2], this solution proposes a server-initiated ACME certificate renewal procedure.

#### 6.x.2.1.1 A server-initiated ACME certificate renewal procedure

Figure 6.x.2.1.1-1 depicts a CA/RA (ACME Server) initiated certificate renewal process. Note that 5G NF acts as an ACME client and the CA/RA is the ACME server.

Figure 6.x.2.1.1 An ACME Server initiated certificate renewal procedure

0. the 5G NF (an ACME client) has been through a certification enrolment process and has received a signed certificate from the CA/RA as described in Solution #5 of 3GPP TR 33.776 v0.4.0 [1].

1. The CA/RA (an ACME Server) is pre-configured with certificate renewal policies and conditions. The conditions may be part of the renewal policies. The certificate renewal may be initiated based on these policies and conditions, for instance:

- Time for renewal (e.g., 11:59 PM EST)

- Remaining valid-time for an issued certificate (e.g., 2/3rd of the valid time).

- AI/ML prediction and data analysis recommendation trigger.

- Security vulnerability or risk condition trigger.

- Scheduled renewal period (e.g., Short-lived certificate).

The server keeps track of these conditions for a client or a set of clients. Furthermore, certificate renewal may be constrained with conditions. For example:

- Renewal window (e.g., between 12:00 AM July 1, 2025 - July 31, 2025)

- Area or segment for renewal (e.g., sector A, geographical region B, 5G Core #5, etc.)

2. The ACME Server may have been pre-configured with a list of ACME Clients that are pre-authorized for certificate renewal. The list may contain the ACME Client identifiers and corresponding security certificates identifiers. The certificate renewal procedure reuses the pre-authorization procedure for certificate issuance described in RFC 8555 [2].

3. The ACME Server detects a trigger for certificate renewal. This trigger to renew the certificate is activated based on policies described in Step 1.

4. The ACME Server checks if an ACME client is pre-authorized for certificate renewal, and if true, sends an indication for certificate renewal. The server also includes a “pre-authorized” indicator to inform that no authorization challenge is required.

5. The ACME Client identifies the certificate renewal request with a ‘pre-authorized’ indicator to learn that no authorization challenge is needed. The ACME client sends a CSR to the server and then downloads the certificate as described in Solution #5 of TR 33.766 [1].

6. If the ACME Client is not pre-authorized, the ACME server sends an authorization challenge [1, 2] along with an indication for certificate renewal.

7. ACME client successfully completes the challenge validation procedure and sends a Certificate Signing Request (CSR) similar to the Steps 3 and 4 in Solution #5 of TR 33.766 [1].

7.a.  If the response from the ACME client indicates that the challenge was not completed successfully, the server sends an indication informing a failure and a new authorization challenge request (go back to Step 6).

Eventually, if the challenge is not completed successfully prior to the expiration time initially provided by the server or updated by the server, the order will be dropped by the server.

6. Provided that the challenge validation is completed successfully the server sends acknowledgement and the client downloads the certificate as described in Steps 5 and Step 6 in Solution #5 of TR 33.766 [1].

### 6.x.3 Evaluation

This solution addresses KI#5.

This solution impacts 5G core network functions.

Pre-defined certificate renewal policies and conditions in the CA/RA trigger the process for certificate renewal.

A 5G NF may be pre-authorized by a CA/RA for certificate renewals. In such a scenario, the CA/RA may not involve challenge validation steps.

If not pre-authorized, the CA/RA sends the certificate renewal indication to a 5G NF. The CA/RA sends challenge objects to the 5G NF which it needs to complete successfully.

The CA/RA may resend a challenge validation request to the 5G NF if the previous challenge was unsuccessful.

Upon receiving a renewal request, the 5G NF completes the challenge and sends a resulting response along with a certificate signing request to the CA/RA. The CA/RA issues to the 5G NF, a signed certificate.

***	END OF CHANGES	***