# 3GPP TSG-SA3 Meeting #118 	S3-243917

# October 14 –18, 2024  Hyderabad, IN

# Source: 	Interdigital

# Title: 	Merger of Solutions #8 and #21

# Document for: 	Approval

Agenda Item: 	5.7

# 1	Decision/action requested

Approve the pCR to TR 33.700-29

# 2	References

[1]	TS 33.501 Security architecture and procedures for 5G System

[2]	TR 33.700-29 Study on Security Aspects of 5G Satellite Access 	in the 5G architecture

# 3	Rationale

This contribution proposes a solution that merges and integrates solutions #8 and #21 of TR 33.700-29.

*** Start of 1st Change ***

6.X			Solution #X: Integration of Solutions #8 and #21 to provide D(DOS) protection from both, pre-provisioned UEs and unauthenticated UEs

### 6.X.1			Introduction

### There are two distinct approaches to the remediation of (D)DOS attacks on the availability of the equipment on board satellite during S&F operation.

### One approach (Solution #8 in this document) requires a pre-provisioned or established through prior network access (e.g., terrestrial)  security context to authenticate and authorize the UE for NTN access. With this approach, the network equipment on board the satellite cannot authenticate and authorize UEs before the security association is established.

### In another approach (e.g., Solution #21 in this document), all UEs are required to produce proof of work by solving puzzles. This approach requires more effort and resources from the adversarial UEs than from the legitimate ones and does not require a pre-provisioned security context.

### There is a need for solutions that integrate and coordinate the use of existing (D)DOS remediation Solution #8 with Solution #21 to take advantage of the benefits in either approach and to allow the remediation of (D)DoS attacks before the security context is established between the UE and the network.

### 6.X.2			Details

The integrated call flow based on Solution #8 and Solution #21 is presented below. It merges Solution #8 and Solution #21 in the present document thus making it possible to remediate unauthenticated (D)DOS attacks.

Figure 6.X.2-1: Integration of Solution #8 and Solution #21

The steps in Figure 6.X.2-1 are described below.

0.	The network provisions UEs and Satellite with a set of credentials for ECCSI. The credentials include Public Validation Token (PVT), Secret Signing Key (SSK) associated with UE's or Satellite's identity, and KMS Public Authentication Key (KPAK). This step is identical to step 0 in Solution #8

NOTE:	KMS can be a standalone entity or collocared in an existing NF.

0a.	The network equipment on board of the satellite may be provisioned with its S&F Policy. This policy may consist among other values with “puzzles required” or “puzzles not required.” The UE may be provisioned with the “S&F Mode” parameter.

After that, the authentication process of S&F mode is as follows.

PHASE 1. (Service link is available, Feeder link is unavailable)

1. The satellite provides a random number generated by the satellite (SAT.RN) and S&F indicator indicating that the satellite is operating in S&F mode. These may be included in the SIB (System Information Broadcast) message. This step is identical to step 1 in Solution #8

2(1). The UE issues an Attach Request to the eNB/MME on board satellite including S&F Mode parameter

3. 	eNB/MME on board satellite checks if the Feeder link is available. If yes, skip to step 11

4a.	eNB/MME on board satellite analyses the received S&F Mode parameter against S&F Policy

If “puzzles” = .TRUE. and the S&F Policy requires puzzles, proceed to step 5

If “puzzles” = .FALSE. or “puzzles” = NULL (i.e., the S&F Mode parameter is not included in message 2  and the S&F Policy requires puzzles, proceed to step 4b

If S&F Policy does not require puzzles, proceed to step 9a

4b.	eNB/MME on board satellite issues Attach Reject (similar to Cause = “S&F Policy Mismatch”). The UE, upon receipt of this message, is aware of its incompatibility with the S&F requirements of eNB/MME on board satellite

5. eNB/MME on board satellite decides to offer a puzzle to throttle (D)DOS attack

6. eNB/MME on board satellite composes a puzzle

7. eNB/MME on board satellite sends a puzzle to the UE

8. The UE receives the puzzle, solves it, and produces the evidence

9a. The eNB/MME on board satellite issues the Attach Reject message with Re-attach Info, SAT.ID, SAT.Sig parameters. This step is optional. If S&F Policy does not require puzzles eNB/MME on board satellite skips this step.

9b. The UE initiates the attach procedure by transmitting signed Attach Request message. This message consists of the Attach Request message (UE.ID, UE.RN, SAT.RN, and S&F indicator in addition to existing parameters), digital signature, and, optionally, the evidence produced in step 8(5). UE.ID of the message, generated by the UE through ECCSI. UE.ID is the UE's identity associated with ECCSI algorithm, and S&F indicator indicates that the UE will operate in S&F mode.

The satellite checks the validity of the UE by verifying the UE.Sig.

10. eNB/MME on board satellite verifies the evidence from step 8. This step is optional. If S&F Policy does not require puzzles eNB/MME on board satellite skips this step

The rest of the call flow follows steps 3 – 12 from Solution #8 in the present document.

### 6.X.3			Evaluation

This solution addresses the requirement of Key issue #1 and does not require pre-provisioning the UE with satellite-specific parameters.

Further evaluation is FFS.

*** End of 1st Change ***