# 3GPP TSG-SA3 Meeting #118 	S3-243919

# October 14 –18, 2024  Hyderabad, IN

# Source: 	Interdigital

# Title: 	Merger of Solutions #15 and #21

# Document for: 	Approval

Agenda Item: 	5.7

# 1	Decision/action requested

Approve the pCR to TR 33.700-29

# 2	References

[1]	TS 33.501 Security architecture and procedures for 5G System

[2]	TR 33.700-29 Study on Security Aspects of 5G Satellite Access 	in the 5G architecture

# 3	Rationale

This contribution proposes a solution that merges and integrates solutions #15 and #21 of TR 33.700-29.

*** Start of 1st Change ***

6.Z			Solution #Z: Integration of Solutions #9 and #21 to provide D(DOS) protection from both, pre-provisioned UEs and unauthenticated UEs

### 6.Z.1			Introduction

### There are two distinct approaches to the remediation of (D)DOS attacks on the availability of the equipment on board satellite during S&F operation.

### One approach (Solution #15) in this document) requires a pre-provisioned or established through prior network access (e.g., terrestrial)  security context to authenticate and authorize the UE for NTN access. With this approach, the network equipment on board the satellite cannot authenticate and authorize UEs before the security association is established.

### In another approach (e.g., Solution #21 in this document), all UEs are required to produce proof of work by solving puzzles. This approach requires more effort and resources from the adversarial UEs than from the legitimate ones and does not require a pre-provisioned security context.

### There is a need for solutions that integrate and coordinate the use of existing (D)DOS remediation Solution #15 with Solution #21 to take advantage of the benefits in either approach and to allow the remediation of (D)DoS attacks before the security context is established between the UE and the network.

### 6.Z.2			Details

The integrated call flow based on Solutions #15 and #21 is presented below. It merges Solution #15 and Solution #21 in the present document thus making it possible to remediate against unauthenticated (D)DOS attacks.

Figure 6.Z.2-1: Integration of Solution #15 and Solution #21

The steps in Figure 6.Z.2-1 are described below.

0a. eNB on board the satellite receives optional eNB S&F policy provisioning

0b. The UE receives optional UE S&F policy provisioning

1. The UE decides based on the optional UE S&F policy what parameter S&F Mode to send to the eNB on board the satellite in Attach Request to the MME-NT. S&F Mode parameter can be e.g. one or more of the following: “puzzles”, “credentials”, and “puzzles” and “credentials.” The intent of using the S&F Mode parameter is to communicate to the eNB on board the satellite the UE policy regarding the remediation of (D)DOS attacks in S&F.

2. The UE initiates the attach procedure by sending the Attach Request with the included S&F Mode parameter to the MME-NT on board the satellite.

3. eNB/MME-NT on board the satellite checks if the Feeder link is available. If yes, skip to step 11

4a. eNB on board the satellite analyses the received S&F Mode parameter received in step 2 against S&F Policy

If “puzzles” = .TRUE. and the S&F Policy requires puzzles, proceed to step 5

If “puzzles” = .FALSE. or “puzzles” = NULL (i.e., the S&F Mode parameter is not included in message 2  and the S&F Policy requires puzzles, proceed to step 4b

If S&F Policy does not require puzzles, proceed to step 11

4b. eNB on board satellite issues Attach Reject (with Cause = “S&F Policy Mismatch”).

5. eNB/MME-NT on board satellite decides to offer a puzzle to throttle (D)DOS attack

6. eNB/MME-NT on board satellite compose a puzzle

7. eNB/MME-NT on board satellite sends a puzzle to the UE using the Attach Reject message

8. The UE solves the puzzle and produces the evidence

9. The UE sends the evidence obtained in step 8 to eNB/MME-NT on board satellite using Attach Request message

10. eNB/MME-NT on board satellite verifies the evidence from step 9. If S&F Policy does not require puzzles eNB/MME-NT on board satellite skips this step

The rest of the Embodiment call flow (i.e., steps 11 – 20) follows steps 2 – 11 from Solution #15 of TR 33.700-29 [2].

11(2). If the feeder link is unavailable, the MME-NT temporarily stores NAS signalling from the UE. When the feeder link becomes available, the MME-NT forwards NAS signalling to the MME-T, including the S&F indication.

12(3). The MME-T interacts with the HSS to obtain the UE subscription information for initiating the authentication procedure.

13(4). If the UE is authorized to use S&F service operation, the MME-T returns the authentication request to the MME-NT. If the UE is not authorized, the MME-T returns the Attach reject to the MME-NT.

14(5). If the service link is unavailable, the MME-NT temporarily stores NAS signalling from the core network. When the service link becomes available, the MME-NT forwards NAS signalling to the UE, which can be Attach Reject message or an Authentication Request message. If the UE is authorized to use S&F service operation, step #12b(5b) is executed. Otherwise, the step #12a(5a) is performed.

If the UE receives an Attach Reject message, the UE stops the attach procedure and waits to re-initiate the Attach procedure until the satellite can establish the service link and feeder link at the same time. Steps 15-20 are skipped.

If the UE receives an authentication request message, the UE returns an authentication response to the MME-NT.

15(6). When the feeder link becomes available, the MME-NT sends an authentication response to the MME-T.

16(7). The MME-T returns NAS Security Mode Command message.

17(8). When the service link becomes available, the UE performs the NAS SMC procedure.

18(9). When the feeder link becomes available, the MME-NT sends NAS SM Complete message to MME-T.

19(10). The MME-T sends the initial context setup request/attach accept.

20(11). When the service link becomes available, the MME-NT forwards the Attach accept message to the UE.

### 6.Z.3			Evaluation

This solution addresses the requirement of Key Issue #1 and does not require pre-provisioning the UE with satellite-specific parameters.

Further evaluation is FFS.

*** End of 1st Change ***