3GPP TSG-SA3 Meeting #118 	S3-243926

# Hyderabad, India, 14 - 18 October 2024

# Source:	IIT Bombay

# Title:	Update on Key Issue 2 related to security aspects on user consent for energy information exposure

# Document for:	Approval

Agenda Item:	5.16

# 1	Decision/action requested

It is requested to approve the update on Key Issue 2

# 2	References

[1] 			3GPP TR 33.766: "Study on security aspects of energy savings in 5G".

# 3	Rationale

This pCR proposed to approve the updates on Key Issue 2 for the study TR 33.766 [1]. It aims to address issues related to user consent required for energy information exposure at granularity. User consent is a must for any activity that can impact user services, as detailed in TR 33.896 and Annex V of TS 33.501.

# 4	Detailed proposal

*** Start of Change ***

## 6.2	Key Issue #2: Security aspects of exposure of energy related information.

### 6.2.1	Key issue details

The key issue aims to address the security issues related to exposure of energy related information studied in TR 23.700-66 [2].

Authentication and authorization are key aspect of securing exposure of network energy-related data, including consumption, efficiency to external consumers as per KI#1 in TR 23.700-66 [1]. This key issue focus on investigating authorization methods for exposure.

User consent is also required for user-specific energy information exposure.

The key issues assumes that the data will be exposed on a monthly or yearly basis, as exemplified in TS 22.261 [1] clause 6.15a.5.2, such linkability attacks using the consumption information as a side channel can be neglected.

Editor’s note: Further details are FFS.

### 6.2.2	Security threats

Potential security threat:

If energy related information is leaked in transit, sensitive information may be disclosed.

Having no authorization or enough level access control can lead to information leakage to authenticated AF’s.

If user-specific energy-related information is exposed without user consent, it could lead to a compromise of user privacy.

Editor’s note: Further security threats are FFS.

### 6.2.3	Potential security requirements

The exposed energy related information shall be integrity and confidentiality protected.

The producer and consumer of energy related information shall mutually authenticate.

The 5GS should support authorization mechanism for the exposed energy related information.

The 5GS should enable granular level access control to be able to restrict and control the flow of energy related information.

Every NF shall confirm user consent check before any energy related information exposure.

The user consent parameters related to energy information exposure (at granular level) shall be stored in the UDM/UDR as subscription data.

Editor’s note: Further requirements are FFS.

*** End of Change ***