3GPP TSG-SA3 Meeting #118 	S3-243928

Hyderabad, India, 14 - 18 October 2024

# Source:	Indian Institute of Technology Bombay

# Title:	Discussion on security overhead for collecting and exposing energy information

# Document for:	Endorsement

Agenda Item:	5.16

# 1	Decision/action requested

This discussion paper provides justifications for a new key issue or a new study item related to security aspects of energy savings in 5G.

# 2	References

[1]	3GPP TR 23.700-66: "Study on Energy Efficiency and Energy Saving (Release 19)".

[2]	3GPP TR 33.766: "Study on security aspects of energy savings in 5G (Release 19)".

[3]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1(Release 19)".

[4]	3GPP TS 28.554: "5G end to end Key Performance Indicators (KPI) (Release 19)".

[5]	3GPP TS 28.318: "Management and Orchestration; Network and services operations for energy utilities ".

[6]	3GPP TS 22.882: "Study on Energy Efficiency as a service criterion (Release 19)”.

# 3	Abbreviations

5GS	5G System

AF	Application Function

OAM	Operations, Administration, and Maintenance

SBI	Service Based Interface

TS	Technical Specification

NEF	Network Exposure Function

NWDAF	Network Data Analytics Function

QoS	Quality of Service

# 4	Rationale

This paper provides insights into the security overhead due to enhancements in 5G for energy saving for collecting and exposing energy information in the network and also with user and application service providers.

# 5	Detailed Discussion

5.1 Motivation

For energy-saving purposes, many 3GPP works have recently emphasized energy awareness in the network by collecting energy-related information, exposing it securely to users/third parties (application service providers), and further adjusting QoS based on energy information [1, 2]. Here, energy information includes energy usage information for network functions, per UE, per service, along with the ratio of the type of energy resources used. This information can also be shared with third parties (application providers). This collection and exposure of energy information within the network and with third parties will require a considerable amount of data exchange, which may lead to increased security overhead. Further, it can impact the overall energy consumption of the network. In this discussion, we highlight the need for a detailed evaluation of all energy-related information parameters from the perspective of security needs. So, if some energy information parameter does not need to be highly secured, we can save security overhead related to its collection and exposure.

Motivation - requirements (from TS 22.261[3])

Subject to user consent and operator policy, 5G system shall be able to provide means to modify a communication service based on energy related information criteria based on subscription policies.

The 5G system shall support different energy states of network elements and network functions.

5G system shall support dynamic changes of energy states of network elements and network functions.

Energy consumption can be monitored and considered through O&M as part of network operations [4, 5], as well as a service criterion.

Subject to operator policy and agreement with 3rd party, the 5G system shall provide a mechanism to support the selection of an application server based on energy related information associated with a set of application servers.

Subject to operator's policy, the 5G network shall support energy consumption monitoring at per network slice and per subscriber granularity.

NOTE 1:	Energy consumption monitoring as described in the preceding requirement is done by means of averaging or applying a statistical model. The requirement does not imply that some form of 'real time' monitoring is required. The granularity of the subscription policies can either apply to the subscriber (all services), or to particular services.

Subject to operator’s policy and agreement with 3rd party, the 5G system shall be able to monitor energy consumption for serving this 3rd party.

NOTE 2: The granularity of energy consumption measurement could vary according to different situations, for example, when several services share a same network slice, etc.

NOTE 3: The energy consumption information can be related to the network resources of network slice, NPNs, etc.

Subject to operator policy and regulatory requirements, the 5G system shall be able to monitor the energy consumption for serving the 3rd party, together with the network performance statistic information for the services provided by that network, related to same time interval e.g., hourly or daily.

NOTE 4: The network performance statistic information could be the data rate, packet delay and packet loss, etc.

5.3 Energy information collection and exposure

In the 5G network, there is a provision for collecting and analyzing energy-related information from the NFs, as detailed in [4, 5]. Energy-related KPIs cover a significant part of end-to-end Key Performance Indicators (KPIs) for the 5G network KPIs [4]. Existing energy-related KPIs in [4] are as follows:

NG-RAN data Energy Efficiency (EE), Generic Network Slice Energy Efficiency (EE) KPI, Energy efficiency of eMBB network slice, Energy efficiency of eMBB network slice – RAN-based, Energy efficiency of URLLC network slice (Based on latency of the network slice, Based on both latency and Data Volume (DV) of the network slice), Energy efficiency of MIoT network slice (Based on the number of registered subscribers of the network slice, Based on the number of active UEs in the network slice), Estimated Virtualized Network Function (VNF) energy consumption, Estimated Virtualized Network Function Component (VNFC) energy consumption, Estimated virtual compute resource instance energy consumption based on mean vCPU usage, Estimated virtual compute resource instance energy consumption based on mean vMemory usage, Estimated virtual compute resource instance energy consumption based on mean vDisk usage, Estimated virtual compute resource instance energy consumption based on I/O traffic volume, 5GC Energy Consumption (EC), Network Slice Energy Consumption (EC), NG-RAN Energy Consumption (EC), gNB EC, Generic 5GC Energy Efficiency (EE) KPI, Energy Efficiency of 5GC based on the useful output of 5GC user plane

In addition to the above existing energy-related KPIs, there is a possibility for including new energy-related KPIs in future specifications as per the latest requirements and objectives (detailed in [1]). Other than energy-related information, network functions may need to share additional relevant information to support energy calculation at a granular level. Some of these information parameters are detailed in Solution#12 of [1]. Energy-related parameters need to be exchanged in the network for energy-aware network decisions (for example, policy updates for adjusting QoS for services) as per the service requirements. Some parts of energy information can also be exposed to third parties and users for some use cases [6].

5.4 Gaps in previously suggested key issues/solutions in ongoing study item [2]

1. Key issues covered in [2] mainly highlight a secure collection of energy information within the network and secure exposure of this information through NEF to the AF. It does not consider concerns about increased security overhead due to this additional data transfer to serve energy related information collection and exposure.

2. Solutions covered in [2] propose to use all existing mechanisms which have the same standard of security for all internal network data. However, there can be varying security requirements for energy-related data, which are not considered in the solution.

5. 5 Questions which need discussion and perspective solutions?

We understand that new requirements related to energy information collection and exposure are included with the aim of energy saving. It's important to discuss what factors can contribute to increasing energy consumption in fulfilling these new requirements and to find solutions to minimize them. In this context, we want to highlight some questions for consideration in the forum:

Should we discuss reducing security overhead in SA3 in this direction, as this is one of the factors that can also contribute to increasing energy consumption?

Should we use the same existing security mechanism to secure energy information exchange, or can there be a new mechanism that can satisfy its security needs, minimize security overhead, and also save energy?

From a broader perspective, can differentiated security mechanisms be applied to information exchange on existing interfaces to save energy?

5.6 Proposal

IIT Bombay proposes to consider one of the following options based on endorsement/discussion:

Option 1: To have a new key issue in ongoing study in SA3 [3] on “Minimization of security overhead by categorizing energy information elements based on their security needs and associated threats”.

Option 2:  To start a new study item on “Minimization of security overhead due to information collection and exposure including energy information”.