3GPP TSG-SA3 Meeting #118	S3-243929

Hyderabad, India, 14th – 19th October 2024

# Source:	MITRE-FFRDC

# Title:	Solution #6 Updates

# Document for:	Approval

Agenda Item:	5.1

# 1	Decision/action requested

This pCR proposes conclusions for TR 33.794[1] solution #6

# 2	References

[1]	3GPP TS 23.501: “System architecture for the 5G System (5GS); Stage 2”

# 3	Rationale

Solution#6 uses DCCF from 3GPP TS 23.288 [x].  The solution provides a starting framework that needs to be enhanced to collect data that can be used for security analytics.  The enhancements for 3GPP TS 23.288 [x] are the responsibility of 3GPP SA2 and it is for SA2 to confirm if 3GPP TS 23.288 [x] is a viable option, and if it is a viable option, SA2 would need to do any enhancements.  This is captured in the conclusions part of solution #6.

Other changes:

Section 7.6.1

Logic for OSF PDP is outside scope of 3GPP, this has been moved from Editors note to a NOTE.

Section 7.6.2.2

Editors note statement is correct, editor’s note has been deleted and its content has been moved to the conclusions section where this is now captured.

Editor’s note on how an OSF knows what data needs to be collected has been identified as being outside scope of any Release-19 work and converted to a NOTE.

Section 7.6.2.4

Editors note has been converted to a NOTE and qualified that this is SA2’s responsibility.

# 4	Detailed changes to TR 33.794 [1]

********************** Start of 1st Change ****************************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 33.894, 2023 September, V18.0.0: "Study on applicability of the zero trust security principles in mobile networks", Release 18.

[3]	3GPP SP-231784, "New Study on enablers for Zero Trust Security".

[4]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[5]	RFC 6749, "The OAuth 2.0 Authorization Framework".

[6]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[7]	3GPP TR 33.894, 2023 September, V18.0.0: "Study on applicability of the zero trust security principles in mobile networks", Release 18.

[8]	NIST Special Publication 800-207: "Zero Trust Architecture".

[9]	3GPP TR 33.738: "Study on security aspects of enablers for network automation for the 5G system phase 3".

[10]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[11]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[12]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[13]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[14]	IETF RFC 9113: "HTTP/2".

[15]	3GPP TS 33.117: "Catalogue of general security assurance requirements"

[16]	3GPP TR 33.926: "Security Assurance Specification (SCAS) threats and critical assets in 3GPP network product classes

[17]	https://owasp.org/www-community/Threat_Modeling_Process

[18]	3GPP TS 23.501: " System architecture for the 5G System (5GS)".

[19]	NIST SP-800-92: "Guide to Computer Security Log Management".

[20]	3GPP TS 29.510: "5G System; Network function repository services; Stage 3".

[21]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[22]	O-RAN.WG11.SecReqSpecs.0-R003-v09.00 "Security Requirements and Controls Specifications"

[AA]	3GPP TS 29.552: "5G System; Network Data Analytics signalling flows".

********************** End of 1st Change ****************************

********************** Start of 2nd Change ****************************

## 7.6	Solution #6: Data Collection using DCCF

### 7.6.1	Introduction

This solution addresses KI#1 Data exposure for security evaluation and monitoring. Specifically, it addresses the requirement: (2) Architecture to be used for exposure of data collected for security evaluation and monitoring of the 5G SBA.

This solution defines the security architecture elements that function as the Security Data Points of Ingest (SDPIs) which can be used by the operator’s security function (OSF) to perform the role of policy decision points (PDPs). SDPIs, in this proposal, are logical entities that enable NFs to provide security data (see sub-clause 7.6.2.4). When an SDPI is implemented as

a)	a sidecar on the NF, the SDPIs can read all SBI security data on the NF. SDPI then registers with an NRF as described in clause 7.3;

b)	a subfunction of an NF, the NF profile is updated to include that the NF can provide security data.

NOTE 1:	In b) it is assumed that the NF providing the security data is secure.  This is an operational / deployment decision.

Security data is to be sent securely using the framework described in 3GPP TS 23.288 [13] which then delivers this data to the OSF.

The OSF remains outside the scope of 3GPP but is expected to provide the logic of PDP and takes the role of a "Data Consumer" as defined in 3GPP TS 23.288 [13].

NOTE 2:	The PDP logic is out of 3GPP scope.

NOTE 3: 	The export of security data from NF to OSF can be done over a secure interface similar to the interface between NEF and external AF (as in TS 33.501 Clause 12.3). Alternatively, part of the OSF (responsible for data reception) can be deployed within the 3GPP network while the security analysis part/logic of OSF is external to 3GPP. In the latter case, a service-based interface can be used between the 3GPP NF and the OSF.

### 7.6.2	Solution details

#### 7.6.2.1	NF profile updates

When an NF supports SDPI functionality, an NFs profile can be updated to indicate support for SDPI and what security data that SDPI is configured to collect.  The NF can register this information in the NRF per 3GPP TS 23.501 [13].

The DCCF profile may need updating to indicate support for collecting security data.

#### 7.6.2.2	Data Collection Configuration

Figure 7.6.2.2-1: Data analytics information request

1. The OSF (i.e. Data Consumer) sends a Ndccf_DataManagment_Subscribe message requesting security data (see sub-clause 7.X.2.4).

2a/b/c	The 3GPP TS 23.288 [13] framework / DCCF sends messages to NFs (i.e. Data sources) (e.g. see 3GPP TS 23.288 [13] sub-clause 6.2.6.3.2) requesting security data (see sub-clause 7.X.2.4).

NOTE 1:	How the OSF as a non3GPP NF know which data needs to be collected is FFS.

NOTE 2:	3GPP TS 23.228 [13] allows for OA&M data (e.g. security logs, counters etc) to be collected, see 3GPP TS 23.288 [13] sub-clause 6.2.6.3.2 step 8a.

#### 7.6.2.3	Data delivery

Figure 7.6.2.3-1: Data transfer

1a/b/c.	3GPP TS 23.288 [13] framework receives security data (e.g. subscribed event data) (see sub-clause 7.6.2.4) from an NF SDPI (e.g. NF1, Nf2, Nf3) ("Data Sources") using functionality specified in 3GPP TS 23.288 [13] (e.g. sub-clause 6.2.6.3).

2	3GPP TS 23.288 [13] framework DCCF decides, based on functionality specified in 3GPP TS 23.288 [13] (e.g. sub-clause 6.2.6.3), sends security data it has received to the OSF (i.e. Data Consumer).

#### 7.6.2.4	Security data

Security data is the data that an SDPI collects and sends to the OSF. The OSF may provide formatting and/or processing instructions to the 3GPP TS 23.288 [13] framework.

Security data that can be collected from an SDPI is specified in the conclusions section of this document.

NOTE: The formatting and the content of the instructions are FFS and will be decided by SA2.

### 7.6.3	Evaluation

This solution describes how the framework defined in 3GPP TS 23.288 [13] could be used to collect security data from the SBA. The MNO can deploy the data collection functionality described in this solution in combination with existing analytics capabilities or deploy dedicated security data collection entities.

The framework in 3GPP TS 23.288 [13] needs to accommodate:

  - The type of security data requested from the data source (e.g. AMF, SMF that are SDPI enabled etc). This is captured in the conclusions section of this document.

  - The actual security data collected from the data source.

  - Update of the Nnf_EventExposure_Subscribe message to support identification of the security data to be collected. This could be an extension of existing messages or a new SBI message that is common across all NFs.

NOTE 1:	Nnf_EventExposure_Subscribe is a generic API message used in 3GPP TS 23.288 [13] and 3GPP TS 29.552 [AA] to refer to all EventExposure messages. 3GPP TS 29.552 [AA] (sub-clause 5.5.1.1 step 3) illustrates this by reference to all other stage 3 message definitions.

NOTE 2: SA2 is needed to confirm the above assumptions and specify any architecture enhancements in 3GPP TS 23.288 [13].

********************** End of 1st Change ****************************