3GPP TSG-SA3 Meeting #118																S3-244004

Hyderabad, India  14 – 18 October 2024

# Source:	Charter Communications, Nokia, CableLabs

# Title:	Discussion paper on access to local services for Femto

# Document for:	Discussion

Agenda Item:

# 1	Decision/action requested

It is requested to discussion this paper on access to local services for Femto

# 2	References

[1] 					3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	TR 38.799 : Study on additional topological enhancements for NR

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[5]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

[6]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

# 3	Rationale

Recently RAN3 conclusion and recommendation on Femto study in TR 38.799 states that, “In order to support access to local services, NR Femto nodes reuse LADN and edge computing functionality as specified in TS 23.501 and TS 23.548”. SA3 need to study security aspects associated with local access for Femto.

# 4	Detailed proposal

5GC supports the following connectivity models to enable Edge Computing as specified in TS 23.548 sec 4.3

-	Distributed Anchor Point: For a PDU Session, the PSA UPF is in a local site, i.e. close to the UE location. The PSA UPF may be changed e.g. due to UE mobility and using SSC mode 2 or 3.

-	Session Breakout: A PDU Session has a PSA UPF in a central site (C-PSA UPF) and one or more PSA UPF in the local site (L-PSA UPF). The C-PSA UPF provides the IP Anchor Point when UL Classifier is used. The Edge Computing application traffic is selectively diverted to the L-PSA UPF using UL Classifier or multi-homing Branching Point mechanisms. The L-PSA UPF may be changed due to e.g. UE mobility.

-	Multiple PDU Sessions: Edge Computing applications use PDU Session(s) with a PSA UPF(s) in local site(s). The rest of applications use PDU Session(s) with PSA UPF(s) in the central site(s). Any PSA UPF may be changed due to e.g. UE mobility and using SSC mode 3 with multiple PDU Sessions.

URSP rules, for steering the mapping between UE applications and PDU Sessions, can be used for any connectivity model and they are required for the Multiple PDU Sessions model.

These three connectivity models are illustrated in Figure below.

5GC Connectivity Models for Edge Computing

SA3 needs to address the following aspects

- RSN-3 study conclusion does not call out any specific connectivity model described above to be used to enable access to local services  for  Femto. This needs to be addressed by SA groups

- SA3 needs to secure the connection for  access to local services for the Femto. This is currently not addressed.

- While enabling access to local services  for Femto leveraging edge computing aspects, SA3/SA3 LI needs to take into consideration LI responsibilities of operators, enable operators to meet these requirements. This may need special provisions as local services may not be deeply integrated with 5GC.

# 5	Conclusions and proposals

Proposal 1: Based on the observations made in this paper, SA3 to address the security and privacy related aspects identified to enable local access for NR Femto into the Rel-19 study.

Proposal 2: Postpone the study of security and privacy aspects of local access to Rel-20.

NOTE: Proposal 2 can leave out security and/or privacy vulnerabilities open in Rel-19.