3GPP TSG-SA3 Meeting #118							S3-244014

Hyderabad, India 14 - 18 October 2024

Source:	Charter Communications, AT&T, Deutsche Telekom, CableLabs

Title:	Discussion paper for ACME

Document for:	Discussion

Agenda Item:	5.4 Study of ACME for Automated Certificate Management in SBA

# 1	Decision/action requested

It is requested to discuss the discussion paper on why ACME is important to Operators

# 2	References

[1] 		IETF RFC 4210: "Internet X.509 Public Key Infrastructure Certificate Management Protocol (CMP)”.

[2]			3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF) ".

[3]	IETF RFC 8555: "Automatic Certificate Management Environment (ACME)".

[4]	3GPP TS 33.776: “Study of Automatic Certificate Management Environment (ACME) for the Service Based Architecture (SBA)”.

3	Rationale

To provide an Operator perspective as to why CMPv2 and ACME both are necessary for certificate management for the entire 5G ecosystem, Core and RAN.

# 4	Detailed proposal

CMPv2 and ACME are both certificate management protocols, however they have been used vastly differently for different purposes within Operator networks.

#### 4.1 CMPv2 in the RAN and in 5G Core

CMPv2 has been used for some time in Mobile Operator networks and is primarily used for certificate management for the RAN dating back to 3G and originally defined by the IETF in 2005. In some operator deployments this is a closed ecosystem with a dedicated Certificate Authority. Often, a purpose-built CMPv2 Server with clients running on the IPSec terminus points: the Security Gateway in close proximity to the Evolved Packet Core, and eNodeB deployed in the field across an untrusted network. In those systems, CMP is only used to manage certificates of IPSec relying parties on the RAN nodes and nothing else.

RA/CA

base station

The base station obtains the operator-signed certificate on its own public key from RA/CA using CMPv2.

CMPv2

Vendor-signed certificate of base station public key is pre-installed.

Vendor root certificate is pre-installed.

SEG

Operator root certificate is pre-installed.

Enrolled base station certificate is used in IKE/IPsec.

IPsec

Figure 7: Overview of the security architecture [2]

These systems are designed to work with static deployments with configuration necessary on both Client and Server for the system to work. It is not automated and requires human interaction.

In Release 18 SA3 studied the usage of CMPv2 as part of certificate management in the 5G Core SBA, as described in 3GPP TR 33.876. The outcome from this study was normative protocols and corresponding procedures for certificate enrolment and renewal for 5G Core Network Functions based on CMP protocol as well as other aspects of certificate management, as reflected in 3GPP TS 33.310 and summarized below.

Clause 10: Certificate Management for 5GC NFs:

- Set up of initial trust – 3 mechanisms are proposed, which share the same design principle, i.e., OAM system is trusted for the operator CA/RA and provides the NF with the initial trust (certificate, IAK or signature) as part of the initial configuration.

- Enrolment and renewal procedures based on a CMPv2 profiling.

- Validation of usage of X.509 certificate (new optional feature)

- Revocation procedures (re-use of existing schemas: CRL, OCSP, OCSP stapling)

- Certificate lifecycle management (new optional requirement)

Annex I (informative): Guidance for 5GC certificates management procedures left to implementation.

These specifications and the informative annex provide options for certificate management based on CMPv2 within the 5G Core SBA.

#### 4.2 ACME

ACME was introduced by the IETF to automate the handling of certificates in web servers in 2019. This is to enable, (i) certification authorities (CAs) in the Web PKI are trusted to verify that an applicant for a certificate legitimately represents the domain name/host  in the certificate, (ii) automate the process of verification and certificate issuance and (iii) facilitate certificate management functions, such as certificate revocation.

There has been a shift in software development to highly virtualized Service Based Architecture and because of that ACME was developed to handle the certificate life cycle for those systems. As part of this shift Operators have deployed SBA throughout the enterprise, their business support systems and operational support systems. The 5G Core in recent releases is the latest domain to include SBA. Operators have built robust ecosystems to manage those systems. These systems are highly virtualized and require certificates for Mutual TLS. These systems are often short lived, typically days but sometimes only minutes.  Due to the highly virtualized systems, manually creating and deploying entity certificates is not feasible, so Certificate Authority’s have been deployed along with a built-in ACME server to automate this function. 5G core has also need to support virtualization of core functions, on demand purpose driven instantiation/de-instantiation of these functions (e.g. 5G core functions to serve certain network slices etc.). Therefore, there is a need to extend rapid trust bestowments capability into the 5G core to bring trustworthiness operational alignment across the more dynamic infrastructure.

ACME Server/CA

NF

2. ACME client creates an account

3. ACME client creates a certificate order for the domain name

1. NF starts the ACME client

5.  ACME client sends the CSR to the server

Standard procedures from RFC 8555

4. ACME client completes the authorizations (http-01/dns-01) of the order

6.  CA issues the certificate

7.  ACME client download the certificate from server

Figure 6.1.2.1.1: ACME procedure for NF certificate management  [4]

# 5	Conclusions and proposals

Based on the observations made in this paper, we are proposing that it is necessary to support both ACME and CMPv2. It should be up to Operator discretion to determine which protocol makes the most sense for their deployment model.