3GPP TSG-SA3 Meeting #118	S3-244021

Hyderabad, India  14 – 18 October 2024

Title:	[DRAFT] Reply to LS on CAPIF phase 3 security considerations

Response to:	LS on CAPIF phase 3 security considerations (S3-243834/S6-243764)

Release:	Rel-19

Source:	3GPP TSG SA3

To:	3GPP TSG SA6

Cc:

Contact person:			Anja Jerichow

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA3 thanks SA6 for their LS on CAPIF phase 3 security considerations and would like to provide the following answers.

SA6 General remark: There several concepts being discussed in SA6 including “user consent”, “resource owner consent”, “resource owner authorization”, SA6 would like to ask SA3 whether they consider them to be equivalent concepts?

SA3 response: The first two terms are equivalent. The latter can assume itself two different meaning. The first is the authorization provided by the resource owner to another entity, this is equivalent to the first two terms. The second meaning, concern what the resource owner is entitled to do. The context should unequivocally identify the correct meaning used for “resource owner authorization”.

SA6 TR 23.700-22 Key Issue #1: Whether SA3 would consider supporting enhancements to assist an AEF in obtaining the purpose of data processing before data processing commences in Rel-19?

SA3 response: The AEF processes access token issued by CCF to the API invoker based on the purpose of data processing. Enhancements to include purpose of data processing will be included in the Authorization request from API invoker and Authorization Function over CAPIF-1/1e and further Authorization function can present such information to Resource owner in the CAPIF-8 interactions to obtain user consent.

SA6 TR 23.700-22 Key Issue #2: Whether SA3 would consider providing the security aspects related to address Single sign-on (SSO) in Rel-19?

SA3 response: Yes, SA3 would consider addressing security aspects in TR 33.700-22 KI#5: Authenticate multiple API Invokers of the same RO (Resource Owner).

SA6 TR 23.700-22 Key Issue #4: Whether SA3 would consider addressing CAPIF interconnection security aspects for AEF service API(s) exposed via CAPIF-6/6e in Rel-19?

SA3 response: Yes, SA3 would consider addressing security aspects in TR 33.700-22 KI#2 and will evaluate the solutions from security perspective.

SA6 TR 23.700-22 Key Issue #6: Whether SA3 would consider lifting the restriction that a UE-deployed API invoker can only access resources owned by the user of that UE.

SA3 response: Yes, SA3 would consider addressing security aspects in TR 33.700-22 KI#3 and identify suitable security solutions.

SA6 TR 23.700-22 Solution #1: Whether SA3 would consider supporting the BFF patten in Rel-19?

SA3 response: No, SA3 has analysed the BFF pattern and the associated IETF draft. As the work in IETF is in draft stage, it is not possible to analyse and conclude such solutions in Rel.19.

SA6 TR 23.700-22 Solution #2: Whether SA3 would consider supporting a security mechanism to enforce user consent checking at an AEF in Rel-19?

SA3 response: Yes, in CAPIF RNAA, the user consent checking at AEF is based on access token which is already supported from Rel.18 and will continue to be supported in Rel.19.

SA3 would like to refer SA6 to the current status of security work related to RNAA in TR 33.700-22.

# 2	Actions

To 3GPP TSG SA

ACTION: 	3GPP TSG SA3 asks 3GPP TSG SA6 to take the above information into account.

# 3	Dates of next TSG SA WG 3 meetings

SA3#119	11 - 15 November 2024		Orlando (US)

SA3#120	17 – 21 February 2025		Athens (Greece)