3GPP TSG-SA3 Meeting #118	S3-244022

Hyderabad, India  14 – 18 October 2024

# Source:	Nokia

# Title:	KI2 Solution on security method retrieval in CAPIF interconnect

# Document for:	Approval, Information, Discussion

Agenda Item:	5.19

# 1	Decision/action requested

Solution on security method retrieval in CAPIF interconnect

# 2	References

[1]	3GPP TS 23.700-22

[2]	3GPP TS 33.700-22

# 3	Rationale

KI#2 solution on CAPIF interconnect needed. Aspect of security method retrieval.

# 4	Detailed proposal

**********   START OF CHANGES

## B.X	Solution #X: Security method retrieval in CAPIF interconnect

### B.X.1	Introduction

This solution is addressing KI#2 on security aspects for CAPIF interconnect, specifically the security method retrieval to allow for authentication and authorization of the API invoker to the AEF.

An API invoker can fetch from CCF (via CAPIF-1/1e) information about the security method to access the API service (via CAPIF-2/2e). An API invoker can also request an API Service and the AEF is fetching the information about the API invoker from CCF (via CAPIF-3).

While this is specified for one security domain, in case of interconnect additional steps are needed because the API invoker is registered with one service provider's CCF while the AEF offering the API services is registered in another service provider’s CCF.

The solution describes how the API invoker is retrieving the supported security method of the exposing API in the CAPIF interconnect case, i.e. the API invoker wants to invoke the API from a different service provider than an AEF known at its local CCF.

### B.X.2	Solution details

#### B.X.2.1 Summary

The API invoker wants to consume APIs offered at an AEF outside of its trust domain. In its Security Method Request message to its local CCF (CCF-B) (via CAPIF-1/1e) it needs to include AEF details along with the security capability information to allow the local CCF to find the CCF where the AEF is registered. [With the current CAPIF framework the API invoker cannot get details from its own CCF (CCF-B) about the APIs offered at the requested AEF in the other security domain.]

Based on the AEF details received from the API invoker, CCF-B identifies the CCF responsible (i.e. CCF-A) where the AEF is registered and forwards the request to this CCF-A.

CCF-A selects a security method for the API service requested from its AEF, considering the information received from CCF-B, access scenarios and AEF capabilities. CCF-A sends a Security Method Response message back to CCF-B, providing the selected security method that allows to get authorization to access the AEF, and any security information related to the security method.

CCF-B sends the response to the API Invoker.

#### B.X.2.2 Information flow

Figure B.X.2.2-1: Information flow to retrieve security method in interconnect

Step 1: Mutual authentication based on client and server certificates shall be established using TLS between the API invoker and the CCF-B.

Step 2: The API invoker sends its CAPIF-2/2e security capability information along with AEF details to the CCF-B in the Security Method Request message.  API Invoker security capability information is to indicate the list of security methods it supports and the AEF details from which the security method information retrieval is requested.

Step 3: CCF-B identifies the CCF (CCF-A) associated with AEF from its locally stored data.

Step 4: CCF-B sends the request to CCF-A, optionally appending CCF-B information.

Step 5: CCF-A shall select a security method to be used over CAPIF-2/2e reference point for requested AEF, considering the information received from CCF-B, access scenarios and AEF capabilities.

Step 6: The CCF-A shall send Security Method Response message to the CCF-B (optionally adding CCF-A information), indicating the selected security method for the AEF, any security information related to the security method.

Step 7: The CCF-B sends the Security Method Response to APIInvoker. Additionally, CCF-B also provides an indication to APIInvoker that AEF belongs to a different domain.

### B.X.3	Evaluation

From key issue KI#2 CAPIF interconnect the aspect security method retrieval to allow for authentication and authorization of the API invoker to the AEFis addressed.

Additional communication between CCFs of different domains is needed.

**********   END OF CHANGES