3GPP TSG-SA3 Meeting #118	S3-244090

Hyderabad, India, 14th - 18th Oct 2024

# Source:	Huawei, HiSilicon

# Title:	Update and evaluation on solution 24

# Document for:	Approval

Agenda Item:	5.10

1	Decision/action requested

Approve the pCR to TR 33.700-32

2	References

[1]	3GPP TR 33.700-32, ' Study on security aspects of User Identities and Authentication’.

3	Rationale

This contribution adds clarifications in some solution’s steps and bring a correction in a figure.

This contribution adds further evaluation to solution 24.

4	Detailed proposal

*** Start of 1st Change ***

6.24	Solution #24: User ID privacy protection based on EAP-TLS protocol using pseudonym mechanism

6.24.1	Introduction

This solution addresses the key issue #2. The user’s privacy protection is based on a pseudonym mechanism to protect the User Identifier (ID). The user ID is replaced with a Member ID (MID) and a different MID is computed each time; hence increases the anonymity and avoid a user ID being tracked. In this solution, the EAP-TLS protocol is used to illustrate how the MID can replace the user ID to achieve anonymity.

6.24.1	Solution details

Aiming at the problem of identity information leakage during the authentication procedure, the solution introduces a new pseudonym mechanism. Users’IDs are grouped in advance and assigned Group User ID (GID ) to the UE. In the identity authentication phase, the EAP-TLS uses Member ID (MID) to replace the unique identity of the user and sent MID to the authentication server (AAA). In order to preserve the privacy of the unique identifier of the user, the AAA utilizes the GID to determine the users’ IDs group and the MID to determine the user's identifier.

Figure 6.24.1 Illustration of user ID privacy protection based on pseudonym mechanism.

0. Provisioning: before the user authentication procedure, the authentication server (e.g., AAA) and the UE are provisioned with the Group User ID (GID). Users’IDs are grouped in advance in a GID, and the UE is allowed to use the User Id and hence GID.

1. After Primary authentication, the UE sends to the SMF a User Activation Request message including the GID, a random number NID, the Member ID (MID). The UE uses the user ID, NID and GID as input to perform a hash operation to obtain the Member ID (MID).

NOTE: The purpose of the random number NID is to ensure the UE calculates a different MID each time; hence it increases the anonymity and avoids User ID being tracked or linked to previous authentication.

2. The SMF initiates the user authentication and sends to AAA the Authentication Request, including MID, NID and GID.

3. The AAA determines the users’ IDs group GID, finds the User ID k of each user in the group, and the AAA calculates MIDK Mwith the NID, User ID k and GID. Then AAA compares whether MID and MIDK  Mare equal. If they are equal, stop querying other User ID k in the group, and the AAA performs the subsequent user identity authentication and authorization based on the user ID. Otherwise, if all users in the group are searched and MID are not equal to MIDK , then the subsequent authentication process will be stopped, and a failure message is generated.

4. After the EAP authentication completes, the AAA sends an EAP-Success message to the SMF, including authentication/authorization result.

5. Based on the result of user authentication (Success/Failure), the SMF returns User Activation Response to the UE, which includes the authentication/authorization result.

6.24.3	Evaluation

This solution addresses the key issue #2.

The solution provides user privacy by using a Member ID (MID) instead of the real user identifier. The privacy of user ID is an end-to-end protected between the UE and the AAA. Since the user identifier is not in the realm of 3GPP systems, how user identifier is mapped to MID is transparent to the 3GPP systems. To prevent tracking or linking of the user, the MID may occasionally be updated to the 3GPP system.

The solution has the following impact at the UE:

The computation of MID by the UE by using the user ID, NID and GID as input to perform a hash operation.

*** End of 1st Change***