3GPP TSG-SA3 Meeting #118	S3-244142

# Hyderabad, India, 14th - 18th October 2024

# Source:	Qualcomm Incorporated

# Title:	Conclusion of KI#1

# Document for:	Approval

Agenda Item:	5.12

# 1	Decision/action requested

This contribution proposes a conclusion of KI #1.

# 2	References

[1]	 TR 33.743

# 3	Rationale

This contribution proposes to add a conclusion of Key Issue #1.

Particularly, it is proposed to reuse the existing security procedure for 5G ProSe UE-to-Network Relay discovery and PC5 security establishment for each hop as described in solution #9,#10, and #x.

# 4	Detailed proposal

It is proposed that SA3 approve the below pCR for inclusion in the TR [1].

***** START OF CHANGES *****

## 7.x	Key issue #1: Security for multi-hop UE-to-Network Relay

The following statements are agreed as a basis for normative work:

For multi-hop UE-to-Network Relay discovery:

- The security procedures for multi-hop UE-to-Network Relay discovery with Model A and Model B are based on solution #9.

For multi-hop UE-to-Network Relay communication:

- The security procedure over User Plane for PC5 security establishment for each hop among 5G ProSe Remote UE, Intermediate UE-to-Network Relay(s), and 5G ProSe UE-to-Network Relay is based on solution #10 and solution #x.

***** END OF CHANGES *****