3GPP TSG-SA3 Meeting #118	S3-244156

Hyderabad, India 14 – 18 October 2024

Title:	LS on 3GPP ReplyLS on PQC Migration

Response to:	LS S3-243812 on LS reply to 3GPP ReplyLS on PQC Migration from GSMA PQTN TF

Release:

Work Item:

Source:	SA3

To:	GSMA PQTN TF

Cc:

Contact person:	Stawros Orkopoulos

stawros dot orkopoulos at nokia dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

# 1	Overall description

SA3 thanks GSMA PQTN TF for their LS S3-243812 on the PQC migration.

SA3 would like to provide responses to the following questions:

Q1: Regarding “SA3 considers that all the 128-bit algorithms currently used in 5G systems can be considered as secure against attacks from both classical and quantum computing.”, As it is known, the NIST was endorsed the AES-128, quoted “remain secure for decades” [2]. For SNOW-128 and ZUC-128 algorithms, is there any endorsement or analysis that SA3 could provide as the reference to prove these algorithms can be considered quantum-safe?

Answer to Q1:

The NIST US Government ([1]) is explicit endorsing the usage of AES-128, while Snow and ZUC are not mentioned at all, while the NCSC UK Government ([2]) is endorsing the usage of 128-bit, while AES-128 is taken as an example. Although the explicit endorsement of Snow and ZUC is missing, SA3 is assuming that the endorsement is implicit applicable to all 128-bit symmetric algorithms which have been specified for the 3GPP Air-link.

Q2: Regarding “3GPP profiles of these protocol”, What exact the 3GPP profiles are? Could SA3 provide some guidance on the 3GPP profiles to prioritize the protocols to be migrated/mandatory.

Answer to Q2:

A 3GPP profile is basically when the configuration parameters of a protocol are adapted according to the needs and for the usage within 3GPP. This adaptation refers to the specification of the parameters and their ranges, as well the selection of options if any exist.

For the prioritisation of the protocols to be migrated, SA3 kindly refers to the cryptographic inventory as published by S3-242820.

# 2	Actions

To GSMA PQTN TF

ACTION: 	SA3 kindly asks GSMA PQTN TF to take the above information into consideration.

# 3	Dates of next TSG SA WG 3 meetings

SA3#119	11 - 15 November 2024		Orlando (US)

SA3#120	17 – 21 February 2025		Athens (Greece)