3GPP TSG-SA3 Meeting #118	S3-244158

Hyderabad, India  14 – 18 October 2024

# Source:	Ericsson

# Title:	New solution for CAPIF interconnection

# Document for:	Approval

Agenda Item:	5.19

# 1	Decision/action requested

It is proposed to add the following solution to the TR 33.700-22.

# 2	References

[1]	3GPP TR 33.700-22: "Study on security aspects of CAPIF Phase3".

# 3	Rationale

This document introduces a solution to TR 33.700-22 [1] for key issue #2 (CAPIF interconnection security).

# 4	Detailed proposal

It is proposed to approve the following change to TR 33.700-22 [1].

***START OF THE 1st CHANGE***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

[XX]	3GPP TR 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

***START OF THE 2nd CHANGE***

### 6.X	Solution #X: CAPIF interconnection

#### 6.X.1 	Introduction

This solution addresses the requirements identified in key issue #2 (CAPIF interconnection security).

It is proposed to re-use the same security mechanisms specified for CAPIF-3/4/5 and CAPIF-3e/4e/5e reference points to address the security aspects of CAPIF-6 and CAPIF-6e reference points, respectively.

For the authentication and authorization of the API invoker by the AEF where the CCF serving the API invoker and serving the AEF are different, it is proposed to include interactions between two CCFs to allow the AEF to learn security and access control policy from the CCF serving the API invoker via the CCF serving the AEF and to allow the CCF serving the API invoker learn information about the AEF via the CCF serving the AEF to be used in the authorization of the API invoker.

#### 6.X.2	Solution details

For CAPIF-6 and CAPIF-6e reference points, same security mechanisms specified in clauses 6.6 and 6.10 of TS 33.122 [XX] for CAPIF-3/4/5 and CAPIF-3e/4e/5e reference points are used, respectively.

For authentication and authorization of the API invoker in the CAPIF-2e reference point, the following figure and steps are the high-level presentation of the solution.

Figure 6.X.2-1: High level solution for authentication and authorization of the API invoker in the CAPIF-2e reference point.

CCF-B and CCF-A are respectively the CCF serving the API invoker and the CCF serving the AEF. It is assumed that the API invoker has been onboarded to CCF-B, the AEF is registered to CCF-A, and the API invoker has discovered service APIs provided by the AEF.

Step 1. The API invoker and CCF-B creates a secure session after successful authentication as specified in TS 33.122 [XX]. If token based authorization is used, then the CCF-B (or CCF-A) issues an access token with the help of CCF-A (or CCF-B).

Step 2. The API invoker interacts with the AEF for service API consumption. Before serving the request, the AEF starts to execute authentication and authorization of the API invoker.

Step 3-6. The AEF learns the security information to be used in the authentication of the API invoker from CCF-B with the help of CCF-A. The security information can be a pre-share key or a root certificate depending on the authentication method. The AEF also learns the access control policy from the CCF-A and CCF-B if token based authorization is not used. In the case of token based authorization, the AEF can learn a root certificate to be used the verification of the access token.

Step 7. The AEF performs the authentication and authorization of the API invoker.

#### 5.X.3	Evaluation

TBD.

***END OF THE CHANGES***