3GPP TSG-SA3 Meeting #118	S3-244310

Hyderabad, India  14th Oct 2024 – 18th Oct 2024

# revision of S3-243869

# Source:	BSI (Federal Office for Information Security)

# Title:	Add specific UDR SCAS test cases for TS 33.530

# Document for:	Approval

Agenda Item:	4.2

# 1	Decision/action requested

This contribution proposes UDR-specific SCAS test cases for the authentication status.

# 2	References

[1]	3GPP TS 33.530 Security Assurance Specification (SCAS) for the Unified Data Repository (UDR) v0.1.0

[2]	3GPP TS 33.117: "Catalogue of general security assurance requirements"

# 3	Rationale

This contribution proposes UDR-specific test cases related to security functional requirements of the UDR to draft TS [1] chapter 4.

These two test cases are derived from the requirements described in Rel-19 TS 33.501, 5.8b Requirements on the UDR and the threat references as part of the Rel-19 TR 33.926 Annex AA. It can be argued that these test cases, much like the requirements and threats, are inherited from the UDM test cases. The UDM can use the UDR as a repository function for storing data. In these cases, the UDR has to work correctly with respect to the requirements. During AKA, the UDR fulfills a critical role in updating the sequence number, but also in storing the authentication status of the UE.

It must be clearly stated that these test cases are security related test cases. Failure of the UDM to meet these requirements for storing the authentication status or updating the authentication data correctly has security implications. For example, failure to update the sequence number can lead to a denial of service in the event of a synchronization failure. If the authentication status is not stored correctly, attacks can bypass the increased home control.

# 4	Detailed proposal

It is proposed that SA3 approve the below changes for inclusion in the TS [1].

********** START OF 1st CHANGE **********

[4]		3GPP TS 33.501: "Security architecture and procedures for 5G system"

[5]		3GPP TR 33.926: "Security Assurance Specification (SCAS) threats and critical assets in 3GPP

network product classes".

[6]	3GPP TS 29.504: “5G; 5G System; Unified Data Repository Services; Stage 3”

********** END OF CHANGE **********

********** START OF 2nd CHANGE **********

## 4.2	UDR-specific adaptations of security functional requirements and related test cases

### 4.2.1	Introduction

The present clause describes the security functional requirements and the corresponding test cases for UDR network product class. The proposed security requirements are classified in two groups:

-	Security functional requirements derived from TS 33.501 [2] and detailed in clause 4.2.2.

-	General security functional requirements which include requirements not already addressed in TS 33.501 [2] but whose support is also important to ensure that UDR conforms to a common security baseline detailed in clause 4.2.2.0.

### 4.2.2	Security functional requirements on the UDR deriving from 3GPP specifications and related test cases

#### 4.2.2.0	General

The general approach in TS 33.117 [3] clause 4.2.2.1 and all the requirements and test cases in TS 33.117 [3] clause 4.2.2.2 related to SBA/SBI aspect apply to the UDR network product class.

#### 4.2.2.1 	Patch authentication data

Requirement Name: Failure to update UE security information

Requirement Reference: TS 33.501 [4], clause 5.8b Requirements on the UDR

Threat References: TR 33.926 [5], clause AA 2.2.1 Failure to store and delete authentication status

Requirement Description: The UDR updates security-related information such as the SQN for resynchronisation.

Test Case:

Test Name: TC_VERIFY_UPDATE_SQN_UDR

Purpose:

- Verify that the UDR under test updated the SQN correctly.

Procedure and execution steps:

Pre-Condition:

- The test case is applicable for the case where UDR and UDM are implemented as separate network functions (non-colocated)

- The tester shall have access to the UE credentials

- The network environment, such as the UDM, can be simulated.

Execution Steps:

1) The tester retrieves the authentication data of the UE from the UDR using the Nudr_DM_Query service.

2) The tester updates the SQN of the UE to a new value at the UDR using the Nudr_DM_Update service and receives a success result from the UDR.

3) The tester retrieves the authentication data of the UE from the UDR using the Nudr_DM_Query service.

Expected Result:

The authentication data from the last request contains the updated SQN.

Expected format of evidence:

Suitable evidence for the interface could include a packet trace, such as a pcap file.

#### 4.2.2.2 	Storing of authentication status of UE

Requirement Name: Storing of authentication status of UE by UDM.

Requirement Reference: TS 33.501 [4], clause 5.8b Requirements on the UDR

Threat References: TR 33.926 [5], clause AA 2.2.2 Failure to store and delete authentication status

Requirement Description: The UDR updates and stores the security relevant data of the UE.

Test Case:

Test Name: TC_PUT_GET_DEL_STATUS_UDR

Purpose:

-	Verify that the UDR under test stores and updates the authentication status of UE.

Procedure and execution steps:

Pre-Condition:

- The test case is applicable for the case where UDR and UDM are implemented as separate network functions (non-colocated)

-	The tester shall have access to the UE credentials

-	The tester is aware that UDR either supports the feature PerUePerSnAuthStatus as defined in TS 29.504 [6] clause 6.1.8 Feature negotiation or not.

- The network environment, such as the UDM, can be simulated.

Execution Steps:

Test 1:

1) The tester stores the authentication status data of a UE in the UDR using the Nudr_DM_Create service

2) The tester retrieves the authentication status of the UE from the UDR using the Nudr_DM_Query service.

3) The tester deletes the authentication status data of the UE in the UDR using the Nudr_DM_Delete service.

4) The tester attempts to retrieve the authentication status of the UE from the UDR using the Nudr_DM_Query service.

Test 2:

NOTE: The following test case is only applicable if the UDR supports PerUePerSnAuthStatus TS 29.504 6.1.8 Feature negotiation.

1) The tester stores the individual authentication status data of a UE in the UDR for a specific serving network using the Nudr_DM_Create service.

2) The tester retrieves the individual authentication status of the UE from the UDR using the Nudr_DM_Query service.

3) The tester deletes the individual authentication status data of the UE in the UDR using the Nudr_DM_Delete service.

4) The tester attempts to retrieve the individual authentication status of the UE from the UDR using the Nudr_DM_Query service.

Expected Results:

For both test cases, the UDM retrieves no (individual) authentication status.

Expected format of evidence:

Suitable evidence for the interface could include a packet trace, such as a pcap file.

********** END OF CHANGE **********