3GPP TSG-SA3 Meeting #118	S3-244316

Hyderabad, India 14 - 18 October, 2024

# Title:	Reply LS on security handling for inter-CU LTM in non-DC cases

# Response to:	S3‑241773/R2-2404037, LS on security handling for inter-CU LTM in non-DC cases

# Release:	Rel-19

# Work Item:	NR_Mob_Ph4-Core

Source:	SA3

To:	RAN WG2

CC:	RAN WG3

Contact person:	Rajavelsamy R

rajvel@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

# 1	Overall description

SA WG3 thanks RAN WG2 for the LS on the security handling for inter-CU LTM in non-DC cases.

SA WG3 has been considering the security mechanisms proposed for the inter-CU LTM feature. For this feature, SA WG3 want to preserve the principle that when there is a PDCP anchor change, then it is necessary to perform a key change (i.e. for solutions or handovers that do not require a PDCP anchor change, there is no need to provide rekeying).

The rekeying method preferred by most companies in SA WG3 is that after every inter-CU LTM cell switch execution that changes the PDCP anchor, the UE is provided with the NCC value via RRC signalling to be used by the UE for key derivation at the next inter-CU LTM cell switch. This solution was preferred as it provides the NCC value using the protected RRC signalling and hence is considered to be secure. SA WG3 understands that, RAN WG2 is aiming to avoid RRC configuration/signalling between cell switches for subsequent LTM procedures. If it is necessary for the RAN WGs to avoid RRC signalling between cell switches, then SA WG3 can analyse further methods like,

- Security threat of the unprotected MAC CE to deliver the NCC value.

- Sending NCC value per candidate cell via RRC message during LTM preparation phase.

SA WG3 will keep RAN WG2 and RAN WG3 informed as we make further progress on this topic.

# 2	Actions

To RAN2:

ACTION: 	SA3 kindly asks RAN WG2 to take the above information into account and provide feedback to SA WG3.

# 3	Dates of next SA WG3 meetings

SA3#119:	11 - 15 November, 2024		Orlando, USA

SA3#120:	17 - 21 February, 2025		Athens, Greece