3GPP TSG-SA3 Meeting #118	S3-244354

Hyderabad, India  14 – 18 October 2024

# Source:	Nokia

# Title:	Solution on authentication aspect in CAPIF interconnect requesting security information from another CCF

# Document for:	Approval, Information, Discussion

Agenda Item:	5.19

# 1	Decision/action requested

Solution on authentication aspect in CAPIF interconnect requesting security information from another CCF

# 2	References

[1]	3GPP TS 23.700-22

[2]	3GPP TS 33.700-22

# 3	Rationale

Solution on authentication aspect in CAPIF interconnect requesting security information from another CCF.

# 4	Detailed proposal

**********   START OF CHANGES

## B.X	Solution #X: Requesting security information from another CCF in order to authenticate using TLS-PSK in CAPIF interconnect

### B.X.1	Introduction

This solution is addressing KI#2 on security aspects for CAPIF interconnect, specifically the authentication aspect between API invoker and AEF if in different security domains.

### B.X.2	Solution details

#### B.X.2.1 Summary

CCF-B and API invoker have obtained the security method that allows to authenticate to the AEF, and any security information related to the security method TLS-PSK. Hence, CCF-B and API invoker can derive AEFPSK based on the AEF’s API service details.

AEF receives an Authentication Initiation Request from APIInvoker, which includes the CCF-B information where the API invoker is registered. AEF requests security information of API invoker from the CCF-A it is registered with, mentioning the APIInvokerID and the CCF-B Information. CCF-A forwards the APIInvokerID to CCF-B which responds to CCF-A with the AEFPSK, which is forwarded to AEF.

API invoker and AEF authenticate using AEFPSK with the knowledge that CCF-B confirmed the APIInvokerID information.

#### B.X.2.2 Information flow

Figure B.X.2.2-1: Information flow to allow authenticating API invoker to AEF in a different security domain

Step 1: APIInvoker gets the AEF details using Obtains_Security method from CCF-B

Step 2: Mutual authentication based on client and server certificates shall be established using TLS between the API invoker and the CCF-B.

Step 3: APIinvoker and CCF-B derives AEF-PSK based on TLS master key used in step 2.

Step 4: APIInvoker sends Authentication Initiation Request to AEF based on AEF details received in step 1 and CCF-B information.

Step 5: AEF requests security information from CCF-A by passing the CCB’s information received in step 4 along with APIInvokerID.

Step 6,7: CCF-A based on CCF-B’s information received requests security information from CCF-B.

Step 8: CCF-B sends the response by providing AEF-PSK to CCF-A.

Step 9: CCF-A sends the response to AEF.

Step 10: AEF sends the Authentication Initiation Response to APIInvoker.

Step 11: TLS connection is established between APIInvoker and AEF using AEF-PSK.

Editor's Note: The need of the APIInvoker to provide the CCF address is ffs.

### B.X.3	Evaluation

TBD

**********   END OF CHANGES