# 3GPP TSG-SA3 Meeting #11 	S3-244394

# October 14 –18, 2024  Hyderabad, IN	was S3-243918

# Source: 	Interdigital

# Title: 	Merger of Solutions #9 and #21

# Document for: 	Approval

Agenda Item: 	5.7

# 1	Decision/action requested

Approve the pCR to TR 33.700-29

# 2	References

[1]	TS 33.501 Security architecture and procedures for 5G System

[2]	TR 33.700-29 Study on Security Aspects of 5G Satellite Access 	in the 5G architecture

# 3	Rationale

This contribution proposes a solution that merges and integrates solutions #9 and #21 of TR 33.700-29.

*** Start of 1st Change ***

6.Y			Solution #Y: Integration of Solutions #9 and #21 to provide D(DOS) protection from both, pre-provisioned UEs and unauthenticated UEs

### 6.Y.1			Introduction

### There are two distinct approaches to the remediation of (D)DOS attacks on the availability of the equipment on board satellite during S&F operation.

### One approach (Solution #9) in this document) requires a pre-provisioned or established through prior network access (e.g., terrestrial)  security context to authenticate and authorize the UE for NTN access. With this approach, the network equipment on board the satellite cannot authenticate and authorize UEs before the security association is established.

### In another approach (e.g., Solution #21 in this document), all UEs are required to produce proof of work by solving puzzles. This approach requires more effort and resources from the adversarial UEs than from the legitimate ones and does not require a pre-provisioned security context.

### There is a need for solutions that integrate and coordinate the use of existing (D)DOS remediation Solution #9 with Solution #21 to take advantage of the benefits in either approach and to allow the remediation of (D)DoS attacks before the security context is established between the UE and the network.

### 6.Y.2			Details

The integrated call flow based on Solutions #9 and #21 is presented below. It combines Solution #9 and Solution #21 in the present document thus making it possible to remediate against unauthenticated (D)DOS attacks.

Figure 6.Y.2-1: Integration of Solution #9 and Solution #21

The steps in Figure 6.Y.2-1 are described below.

0. The network equipment on board the satellite may be provisioned with its S&F Policy. This policy may consist among other values of “puzzles required” or “puzzles not required.” The UE may be provisioned with the “S&F Mode” parameter.

1. Initial Registration Process: Upon entering the satellite's coverage, the UE initiates an Initial Attach Request with the S&F Mode parameter.

2. E-UTRAN on board satellite checks if the Feeder link is available. If yes, the (D)DOS attack on availability is comparable to similar (D)DOS attacks in terrestrial networks, and the network equipment on board the satellite skips to step 11. This is the optimization of Solution #21

3a. eNB on board the satellite analyses the received S&F Mode parameter against S&F Policy

If “puzzles” = .TRUE. and the S&F Policy requires puzzles, proceed to step 5

If “puzzles” = .FALSE. or “puzzles” = NULL (i.e., the S&F Mode parameter is not included in message 2  and the S&F Policy requires puzzles, proceed to step 3b

If S&F Policy does not require puzzles, proceed to step 10

3b. eNB on board satellite issues Attach Reject (with Cause = “S&F Policy Mismatch”)

4. eNB on board satellite decides to offer a puzzle to throttle (D)DOS attacks

5. eNB on board satellite composes a puzzle

6. eNB on board satellite sends a puzzle to the UE

7. The UE solves the puzzle and produces the evidence

8. The UE issues a NAS Attach Request from to the eNB on board satellite with the solved puzzle (evidence)

9. eNB on board satellite verifies the evidence from step 8. This step is optional. If S&F Policy does not require puzzles eNB/MME on board satellite skips this step

The rest of the Embodiment call flow (i.e., steps 9 – 19) follows steps 2 – 12 from Solution #8 of TR 33.700-29 [2].

10-11. The MME-SAT, unable to immediately establish a ground connection, temporarily stores the UE's International Mobile Subscriber Identity (IMSI) and issues an Attach Reject message. The MME-SAT rejects the UE's Initial Attach Request with an Attach Reject message that includes a Cause value indicating that the Attach procedure is suspended, as well as a Timer value(indicating how long the UE should refrain from attempting another Attach).

NOTE: The Cause and the Timer can be protected with a digital signature, which the UE can validate using provisioned root certificates.

12-13. Once MME-SAT establishes contact with MME-GND, it forwards the IMSI to request authentication vectors from the Home Subscriber Server (HSS).

14-18. In subsequent coverage, the UE re-initiates the Attach Request. This time, MME-SAT, equipped with the authentication vectors, proceeds to authenticate the UE, leading to a successful Attach Acceptance. Immediately following successful authentication, MME-SAT sends a provisional Update Location Request to the HSS. This update includes an indicator that the location update is provisional and should not be fully processed until final confirmation is received, optimizing the handling of location data under intermittent connectivity.

19-20   Location Update Process: MME-SAT updates the UE's location with the HSS upon establishing ground connectivity, ensuring the UE's subscription permits service in the attempted location. Any discrepancies trigger a detach procedure during the next satellite contact.

### 6.Y.3			Evaluation

This solution merges Solutions #9 and #21 by “stacking” them up. It addresses the requirement of Key issue #1 and does not require pre-provisioning the UE with satellite-specific parameters while allowing it to serve UEs with pre-provisioned satellite-specific parameters. It will enable skipping DDOS remediation when the Feeder Link is available.

The specific evaluations of Solution #9 and Solution #21 are applicable here.

- Assumptions: Only S&F related DDoS attack which can lead to buffer overflow during S&F is the focus of this solution.

- Dependency on other WGs: S&F Policy provisioning (step 0) and offering puzzles (step 6), would require coordination with RAN.

- Relevant KI and potential security requirements addressed: This solution addresses KI#1 Requirement #3.

- Architecture option: The solution is applicable to split-MME architecture option.

- Re-use of legacy security procedures: None

- Advantages of the solution: This solution does not require pre-provisioning the UE with satellite-specific parameters while reducing the rate of DDOS attacks. On the other hand, this solution allows the authorization of the UEs with pre-provisioned satellite-specific parameters when available. This solution also enables skipping DDOS remediation when the Feeder Link is available.

- Disadvantages of the solution: As inherited from Solutions #9 and 21.

- Impacted entities: UE, Satellite eNB and MME, EPC

Editor’s Note: Further evaluation is FFS.

*** End of 1st Change ***