3GPP TSG-SA3 Meeting #118	S3-244454

Hyderabad, India  14 – 18 October 2024

# Source:	Ericsson

# Title:	Updating KI#5 Authentication in Ambient IoT service

# Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

This document proposes to update KI#5 Authentication in Ambient IoT service.

# 2	References

None

# 3	Rationale

The EN on identifying further threats needs to be addressed. New threats to the existing threats are added. A new security requirement is proposed on the credentials for authentication at the application layer when application layer authentication is required.

# 4	Detailed proposal

**** START OF CHANGE ****

## 5.5	Key Issue #5: Authentication in Ambient IoT service

### 5.5.1	Key issue details

The TR 23.700-13 [4] studies the architecture support of Ambient Internet of Things (AIoT) device, considering the service requirements for ambient power-enabled IoT device. In TR 23.700-13 [4], the validation of the AIoT device identity and authentication are explicitly mentioned.

### 5.5.2	Threats

In the air interface, an attacker may impersonate the victim device and report fake identification to the network side. If the billing is based on per AIoT device’s identity, the fake identity may lead to charging problem. This can be used by an adversary to steal an AIoT device by replacing the AIoT device with a fake device, which might cause a loss to the owner of the device. Whether fake identities pose a threat in case of inventory use case, is up to the individual deployments risk profile.

Over the air interface, without authentication, an adversary can impersonate a legitimate network and send a command — e.g., disablement command will permanently deactivate a device.

Editor’s Note: additional threat is FFS.

### 5.5.3	Potential security requirements

The 5G system shall provide a means to perform mutual authentication between the AIoT device and the network.

NOTE: If solution reuses the existing authentication framework (e.g., 5G-AKA, EAP-AKA’, other EAP methods for SNPN), the existing UE security requirements in TS 33.501[5] apply. Otherwise, the core network entities, if any, that are used for authentication are dedicated for Ambient IoT service.

Editor’s Note: Secure storage and processing of credentials (on the AIoT device and network side) for solution not reusing existing authentication framework as per the note above is ffs.

Editor’s Note: Whether authentication of AIoT device is mandatory is ffs.

Editor’s Note: Whether one-way authentication needs to be defined is ffs.

Editor’s Note: Whether authentication of AIoT device can be done on application layer is ffs.

**** END OF CHANGE ****