3GPP TSG-SA3 Meeting #118 	S3-244484

# Hyderabad, India, 14 - 18 October 2024                                      revision of S3-244177

# Source:	Apple

# Title:	New solution to address key issue#3 on privacy protection

# Document for:	Approval

Agenda Item:	5.6

1	Decision/action requested

Approve the pCR to TR 33.713

- References

- TR 33.713 v0.3.0

3	Rationale

This contribution proposes a new solution to address key issue#3 in TR 33.713.

4	Detailed proposal

*** Start of 1st Change ***

6.X	Solution #X: Privacy protection on AIoT device IDs

6.X.1	Introduction

This solution addresses key issue#3: “Mechanisms for mitigating privacy threats (described above) by identifying, linking, and tracking the identifiers of AIoT Device(s) shall be supported.”

6.X.2	Details

Preassumption: the AIoT device has one ID preconfigured by the manufacture (Application Function).

Step 0a. the device is configured with one Device ID in the manufacturing time. Every ID is configured corresponding

to an Index.

Editor’s Notes: Clarifications of the configuration of device ID and the Index and the mapping is FFS.

Editor’s Notes: How to address the synchronizaiton issue is FFS.

Editor’s Notes: if the ID is sending in the plaintext in paging message, the attacker can track the HASH chain, how to address this issue is FFS.

Editor’s Notes: It is FFS how the procedure is integrated into inventory procedure, e.g. how the Reader report the device ID to the AF, when the inventory procedure is triggered.

Editor’s Notes: It is FFs what is the impact on the device and network.

Step 0b. the Application Function sends the Device IDs and Indexes to the AIoTF through NEF.

Step 0c. AIoTF sends the corresponding Device IDs to each Reader based on distribution policy from Application Function or local policy from MNOs.

Step 0d. Reader stores the Devices IDs and the corresponding Indexes under this Reader.

Step 1. Reader sends the paging message to the AIoT devices. Reader sends ID1-1, ID2-1, … in the paging message.

NOTE 1: According to RAN2 agreement in #116, the paging message may contain one ID (one Device ID or one group ID) or more (multiple IDs FFS in RAN2) IDs. The procedure may be updated based on RAN2 progress.

NOTE 2: IDm-n indicates the n-th ID for Device m.

Step 2: AIoT device replies with ID1-1||Index to the Reader.

NOTE 3: Index shall indicate which ID of which device.

Step 3: Reader compares the ID1-1 with its data base, then confirm this ID1-1 is in its data base.

Step 4: Reader echoes back the ID1-1 to device, following RAN2 procedure.

Step 5: Device checks ID1-1 is correct, use ID1-2 in next message, in which ID1-2 = HASH (ID1-1)

NOTE 4: Reader shall use unused ID for each device for the next paging if there is any.

6.X.3	Evaluation

TBD

*** End of 2nd Change ***