3GPP TSG-SA3 Meeting #118 	S3-244504

# Hyderabad, India, 14 - 18 October 2024	(Revision of S3-243926)

# Source:	IIT Bombay

# Title:	Update on Key Issue 2 related to security aspects on user consent for energy information exposure

# Document for:	Approval

Agenda Item:	5.16

# 1	Decision/action requested

It is requested to approve the update on Key Issue 2

# 2	References

[1] 			3GPP TR 33.766: "Study on security aspects of energy savings in 5G".

# 3	Rationale

This pCR is proposed to approve the updates on Key Issue 2 for the study TR 33.766 [1]. It aims to address issues related to user consent required for energy information exposure at granularity. User consent is a must for any activity that can impact user services, as detailed in TR 33.896 and Annex V of TS 33.501.

The below requirement outlined in TS 22.261 emphasizes the need for obtaining user consent before disclosing energy-related information:
“Subject to operator’s policy, agreement with 3rd party and consent by the customer, the 5G system shall be able to expose the network performance statistic information (e.g. the data rate, packet delay and packet loss) together with energy consumption information resulting from service provided to the customer, to the authorized third party, related to the same time interval e.g. hourly or daily.”

In addition, justification section of approved SID (SP-240512) refers to privacy aspect of user, “By exposing information, which can be linked to a subscriber, there is a potential for a privacy leak. Whether additional measures need to be taken to preserve the privacy of the subscriber needs to be further studied.”

Energy information exposure can take place at various granularities and can be shared with any authorized AF at the granularity consented by the user. Below conclusion on network energy related information exposure, as outlined in clause 8.1 of TR 23.700-66, emphasizes the importance of obtaining user consent for exposing information at various granularities.

“If the authorized consumer is AF, the granularities include: per UE, per UE per application, per PDU session.”

# 4	Detailed proposal

*** Start of Change ***

## 6.2	Key Issue #2: Security aspects of exposure of energy related information.

### 6.2.1	Key issue details

The key issue aims to address the security issues related to exposure of energy related information studied in TR 23.700-66 [2].

Authentication and authorization are key aspect of securing exposure of network energy-related data, including consumption, efficiency to external consumers as per KI#1 in TR 23.700-66 [1]. This key issue focus on investigating authorization methods for exposure.

The key issues assumes that the data will be exposed on a monthly or yearly basis, as exemplified in TS 22.261 [1] clause 6.15a.5.2, such linkability attacks using the consumption information as a side channel can be neglected.

Editor’s Note: As regional legislation to privacy complies, the need of solutions to support compliance is FFS.

Editor’s note: Further details are FFS.

### 6.2.2	Security threats

Potential security threat:

If energy related information is leaked in transit, sensitive information may be disclosed.

Having no authorization or enough level access control can lead to information leakage to authenticated AF’s.

Editor’s note: Further security threats are FFS.

### 6.2.3	Potential security requirements

The exposed energy related information shall be integrity and confidentiality protected.

The producer and consumer of energy related information shall mutually authenticate.

The 5GS should support authorization mechanism for the exposed energy related information.

The 5GS should enable granular level access control to be able to restrict and control the flow of energy related information.

Editor’s note: Further requirements are FFS.

*** End of Change ***