# 3GPP TSG-SA3 Meeting #118 	S3-243854

# October 14 –18, 2024  Hyderabad, IN

# Source: 	Interdigital

# Title: 	An update for Key Issue #2: Authorization for 5G Ambient IoT Services

# Document for: 	Approval

Agenda Item: 	5.9

# 1	Decision/action requested

Approve the pCR to TR 33.713

# 2	References

[1]	TS 33.501 Security architecture and procedures for 5G System

[2]	TR 38.769 Study on solutions for Ambient IoT (Internet of Things) in NR

[3]	TS 22.369 Service requirements for Ambient power-enabled IoT

# 3	Rationale

This contribution proposes to update Key Issue #2: Authorization for 5G Ambient IoT services due to the need to be able to authorize Reader nodes in both, Topology 1 and Topology 2.

# 4	Detailed proposal

SA3 is asked to approve the following pCR.

*** Start of 1st Change ***

## 5.2	Key Issue #2: Authorization for 5G Ambient IoT services

### 5.2.1	Key issue details

In TR 23.700-13 [4], Key Issues #1 and #3 describe the issues on the system architecture and procedure to support 5G Ambient IoT services.

In Topology 2 as defined in TR 38.769 [2], the UE acting as the intermediate node is responsible for transferring the information between the AIoT device and 5GS. If the authorization of the intermediate node is not supported, the attacker can play the role of the intermediate node and arbitrarily deny 5G AIoT service.

Per TR 23.700-13 Clause 6.3.1.4, Readers may be deployed independently of 3GPP UEs, i.e. as stand-alone base stations to enable Topology 1. Such Readers acting as stand-alone base stations, if unauthorized, may mount similar attacks as the unauthorized UEs in Topology 2.

Therefore, it is necessary to study how to authorize the UEs to act as the intermediate nodes and Readers to act as stand-alone base stations.

### 5.2.2	Security threats

If the 5GC cannot verify if the UE acting as an intermediate node is authorized, the attacker UE may impersonate the intermediate node. The attacker UE may then deny the 5G Ambient IoT services.

A competitor or thief can perform an unauthorized inventory of a store by scanning AIoT devices with an unauthorized reader to determine the types and quantities of items. An unauthorized reader can query the AIoT devices for e.g.,  EPC number if AIoT devices used in the supply chain respond to unauthorized UEs acting as intermediate nodes and Readers acting as stand-alone base stations.

.

### 5.2.3	Potential security requirements

The 5GS shall be able to support the authorization of the AIoT capable UEs as an intermediate nodes in 5G Ambient IoT services.

The 5GS shall be able to support the authorization of the Readers acting as stand-alone base stations in 5G Ambient IoT services.

*** End of 1st Change ***