3GPP TSG-SA3 Meeting #118	S3-243940

Hyderabad, India  14 - 18 October 2024

# Source:	Nokia

# Title:	Discussion paper for CT1 LS on SUCI calculation

# Document for:	discussion

Agenda Item:	3

# 1	Decision/action requested

It is requested to discuss the discussion paper on SUCI calculation

# 2	References

[X]

# 3	Rationale

CT1 LS analysis and propose a way forward.

# 4	Detailed proposal

## 4.1 Background

### 4.1.1 CT6 CR- C6-240155

Summary: CR is agreed with the text if information like “SUPI Type”, “HN ID”,  “RID” etc required for the SUCI calculation is not correctly configured in the USIM, the USIM shall return an error status word.

Reference: C6-240155/ 31.102: #### 7.5.1.1 SUCI context ….. For the execution of the command, the following information shall be available in the USIM: - Home network identifier (i.e. MCC and MNC when SUPI Type is IMSI or domain name when SUPI Type is Network Specific Identifier, Global Line Identifier or Global Cable Identifier) (see NOTE). - Routing indicator (configured in EFRouting_Indicator). - Home network public key (see Note). - Home network public key identifier (see Note). - Protection scheme identifier (see Note). - SUPI. …. In case the information required for the SUCI calculation as listed above is not correctly configured in the USIM, the USIM shall return an error status word as described in Section 7.3.2. ………….

### 4.1.2 CT1 LS- C1-245039

Summary: In C6-240155 (CR #1030 for TS 31.102) CT6 clarified that in case the information required for the SUCI calculation is not correctly configured in the USIM, USIM shall return an error status word. However it is unclear how the UE shall behave in this case in order to get service either in 5GC or EPC.

ACTION: CT1 kindly asks SA3 to discuss the topic and provide feedback how the UE should handle the described SUCI calculation failure.

### 4.1.2 Associated problem details

As mentioned in the CT6 CR in 4.1.1 and the LS to SA3 in 4.1.2 above, SUCI computation failure could happen because of multiple reasons. It could be wrong or missing parameter values, either due to misconfiguration or erasure of the parameter from the configuration file in the USIM. For whatever reason, when SUCI computation failure happens in the 5G USIM  an ‘error status word’ is returned to ME. What is expected behaviour of Mobile Equipment in this scenario is not defined in any of the specifications.

Once this error happens, leaving the UE in the same state will cause,

- UE (and user) to not get any service,

- for operators revenue loss will occur,

- if UE is not able to generate the SUCI, then authentication cannot be performed, and UE will not be able to connect to network  to report this error to the network.

This kind of error can occur in multiple scenarios for the UE:

Scenario #1:UE is registered and authenticated in 5GC. NAS and AS has the security context. Home network updates the configuration related to SUCI and now the SUCI cannot be calculated.

- AMF request for Identity request for SUCI but SUCI calculation fails.

Scenario#2: Home network updates the configuration related to SUCI but then UE moves to unregistered state. Therefore, no security context in NAS or in AS is available.

- UE tries to trigger the registration request towards the 5GC but fails because SUCI calculation fails.

Scenario#3: Home network updates the configuration related to SUCI and then UE moves to EPC/4G.

- UE comes back in 5GC but registration to 5GC fails because SUCI calculation fails. So UE can still use the EPC service, not 5GC.

Observation : Somehow UE should be able to come out of the error and inform to 4G/5G network of the exact error or missing parameter so that network can take corrective actions so that UE services in 5G can be resumed, maybe via EPC or emergency registration route. i.e. if UE can still connect to EPC/4G, so UE can report this error to EPC and EPC/5GC can take corrective actions. If UE is only 5G UE, then UE can still perform emergency registration and inform this error to the HPLMN.

# 5	Conclusions and proposals

Based on observations made in this paper, it is proposed that

- Start a mini WID in the next meeting so that we address this issue, companies can bring solutions and send the LS reply accordingly.