3GPP TSG-SA3 Meeting #118	S3-244001

Hyderabad, India 14th – 18th October 2024

Title:	LS on NR Femto GW for aggregating user plane interfaces

Response to:

Release:	Release 19

Work Item:	FS_5G_Femto_Sec

Source:	Charter Communications (to be SA3)

To:	RAN3

Cc:

Contact person:	Achari Kakinada

Achari.Kakinada@Charter.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S3-244002

# 1	Overall description

SA3 observed that, in the recent conclusions from RAN3 study TR 38.799 Study on additional topological enhancements for NR, architecture option-2 only mentions about NR Femto GW acting as a concentrator for control plane traffic only and anonymizing of AMF function from the NR Femtos. There is no mention of concentration of user plane traffic or anonymizing UPF from NR Femtos. This exposes a key function like UPF in operator core network to the NR Femtos potentially deployed in premises beyond operators control and making the operators network vulnerable. This violates the principle of operator network topology hiding from the NR Femtos.

SA3 would like to ask RAN3 to support architecture enhancements for user plane traffic concentration through the NR Femto GW and provide anonymity for UPF to facilitate topology hiding from NR Femtos.

# 2	Actions

To RAN3

ACTION: 	SA3 kindly asks RAN3 to take the above information into consideration and provide further guidance to SA3.

# 3	Dates of next TSG SA WG 3 meetings

SA3#119	11 - 15 November 2024		Orlando (US)

SA3#120	17 – 21 February 2025		Athens (Greece)