3GPP TSG-SA3 Meeting #118	S3-244079

Hyderabad, IN 14 - 18 Oct 2024

# Source:	OPPO

# Title:	Integrity protection of inter-CU LTM Cell Switch Command MAC CE

# Document for:	Approval

Agenda Item:	4.12

1	Decision/action requested

It is proposed a new solution on integrity protection of inter-CU LTM Cell Switch Command MAC CE .

2	References

[1]	TS 33.501 Security architecture and procedures for 5G System

[2]	R2-2404037

3	Rationale

It is proposed to approved this pCR in Living document on NR mobility enhancement.

In LS R2-2404037[2], RAN2 requests SA3 whether, for each option, the change of security algorithm or the change of key set indicator is to be supported for inter-CU LTM. Hence, we analyse the change of security algorithm and the change of key set indicator, as follows:

Analysis of the change of security algorithm:

- As specified in TS 33.501[1], if target gNB supports different algorithms from source gNB, the target gNB shall indicate the chosen algorithm to the UE through source gNB in the HO message. In inter-CU LTM senariao, the security negotiation between target gNB and UE can be performed during the LTM preparation phase, i.e., the source gNB sends the used algorithm with UE and UE’s security capability to the target gNB, therefore, the target gNB sends the chosen algorithm to the source gNB, which can be configured to the UE in the candidate configuration. Or the security negotiation between target gNB and UE can be performed during the LTM excecution phase, and carried by MAC CE.

Analysis of the change of key set indicator:

- As specified in TS 33.501[1], the gNB sets keySetChangeIndicator in the HO command to true, when AS key re-keying procedure is initiated by the AMF. In such case, the intra-gNB-CU could be performed to applied new security contect between source gNB and UE. The change of key set indicator could be performed during the inter-CU LTM procedure, and carried by MAC CE, which should be integrity protected, otherwise, the keySetChangeIndicator field may be tampered by attacker and further lead to handover failure.  Or the change of key set indicator is performed independently, for instance, while the AS key re-keying procedure is initiated by the AMF, the source gNB triggers intra-gNB-CU/intra-ng-eNB handover immediately, before the inter-CU LTM.

In inter-CU LTM senariao, the source gNB triggers the UE to perform handver via MAC CE, which is lack of integrity protection or confidentiality protection. According to R18 intra-CU LTM, the MAC CE includes Target Configuration ID, Timing Advance Command, TCI state ID, UL TCI state ID, etc. In R19, the intre-CU LTM Cell Switch Command MAC CE may include NCC, the algorithms of Target gNB selection, and the change of key set indicator, according to the analysis above. Hence, the UE should be able to verify the recieved MAC CE is genuine, otherwise the inter-CU LTM Cell Switch Command MAC CE could be tampered and further caused handover failure. If the inter-CU LTM Cell Switch Command MAC CE is lack of confidentiality protection, the contained parameters could be eavesdropped by attacker. Considering pertantial threats, the inter-CU LTM Cell Switch Command MAC CE should be integrity protected.

4	Detailed proposal

**** Start of Change****

3.Y	Solution #Y: Integrity protection of inter-CU LTM Cell Switch Command MAC CE

3.Y.1	Introduction

This solution utilizes the exsiting AS security between UE and serving gNB to intrgrity protect the LTM Cell Switch Command carried by MAC CE for inter-CU LTM and subsequent inter-CU LTM senariaos. In this solution, the inter-CU LTM Cell Switch Command MAC CE includes the essential parameter NCC, optional includes chosen algorithms of target gNB and the KeySetChangeIndicator.

3.Y.2	Solution details

The procedure below comprises LTM preparation phase, LTM execution phase, and subsequent LTM execution phase.

Figure 3.Y.2.1: Integrity protection of inter-CU LTM Cell Switch Command MAC CE

- During the LTM preparation phase, the Source gNB triggers the security negotiation between UE and all the candidate Target gNBs by sending UE’s security capabilities and ciphering and integrity algorithms used in the source gNB.

- Each candidate target gNB selects the algorithm with highest priority from the received security capabilities of the UE according to the prioritized locally configured list of algorithms. If the target gNB selects different algorithms compared to the source gNB/ng-eNB, the chosen algorithms should be sent back to the source gNB.

- [Optional] The source gNB configures the chosen algorithms of candidate gNBs to the UE during the LTM preparation phase.

- In the LTM execution phase, the UE sends the L1 measurement report to the source gNB.

- The source gNB determines to LTM handover UE to the target gNB1, and genaretes the KNG-RAN* using the KgNB if horizatal handover performs, or using the fresh NH if there is unused {NH, NCC} pair, and vertical handover performs, as specified in 33.501[2]. Then the source gNB provisions the KNG-RAN* and corresponding NCC to the target gNB1.

- The targrt gNB1 uses the KNG-RAN* as KgNB, and responses ACK to the source gNB.

- The source gNB sends the inter-CU LTM Cell Switch Command MAC CE to the UE, which includes the index of Target gNB1, NCC, and optional includes the chosen algorithm of Target gNB1 and KeySetChangeIndicator. To integrity protect the inter-CU LTM Cell Switch Command MAC CE, the source gNB utilizes the exsiting AS security contects (e.g., KRRCint) of UE to calculate the MIC, and includes the MIC in the inter-CU LTM Cell Switch Command MAC CE.

NOTE 1:	Steps 5 and 7 are performed in parallel, so that the target gNB1 is provisioned with the KNG-RAN* and corresponding NCC before step 9.

- The UE verifies the integrity of received inter-CU LTM Cell Switch Command MAC CE using the KRRCint.

- The UE sends the RRC configuration complete to the target gNB1.

- The target gNB1 gNB sends the path switch request to the AMF.

- The AMF sends the path switch response to the source gNB, including the unused {NH, NCC} pair.

- [Optional] If AMF triggers AS key re-keying, an additional RRC message could be sent from the serving gNB to the UE, which is separated from the inter-CU LTM.

The subsequent LTM procedure repeats from step 4 to step 11.

3.Y.3	Evaluation

**** End of Change****