3GPP TSG-SA3 Meeting #118	S3-244098

Hyderabad, IN, 14 – 18 October 2024

# Source:	Nokia

# Title:	New key issue for Security and privacy aspects of local access

# Document for:	Approval

Agenda Item:	5.17

# 1	Decision/action requested

SA3 is kindly requested to approve the proposed key issue.

# 2	References

# 3	Rationale

This contribution proposes a new key issue for studying security and privacy aspects of local access. As per conclusions in RAN TR 38.799, local access is supported in Rel-19. SA3 needs to study the security aspects of local access.

# 4	Detailed proposal

******** FIRST CHANGE ********

## 5.X	Key Issue #X: Security and privacy aspects of local access

### 5.X.1	Key issue details

RAN3 conclusion and recommendation on Femto study in TR 38.799 V19.0.0 (2024-09) states that, “In order to support access to local services, NR Femto nodes reuse LADN and edge computing functionality as specified in TS 23.501 and TS 23.548”. SA3 need to study security aspects associated with local access for Femto.

As per TS 23.548 clause 4.3 elaborates following connectivity models to enable Edge Computing:

- Distributed Anchor Point: For a PDU Session, the PSA UPF is in a local site, i.e. close to the UE location. The PSA UPF may be changed e.g. due to UE mobility and using SSC mode 2 or 3.

- Session Breakout: A PDU Session has a PSA UPF in a central site (C-PSA UPF) and one or more PSA UPF in the local site (L-PSA UPF). The C-PSA UPF provides the IP Anchor Point when UL Classifier is used. The Edge Computing application traffic is selectively diverted to the L-PSA UPF using UL Classifier or multi-homing Branching Point mechanisms. The L-PSA UPF may be changed due to e.g. UE mobility.

- Multiple PDU Sessions: Edge Computing applications use PDU Session(s) with a PSA UPF(s) in local site(s). The rest of applications use PDU Session(s) with PSA UPF(s) in the central site(s). Any PSA UPF may be changed due to e.g. UE mobility and using SSC mode 3 with multiple PDU Sessions.

URSP rules, for steering the mapping between UE applications and PDU Sessions, can be used for any connectivity model and they are required for the Multiple PDU Sessions model.

For these connectivity models, SA3 needs to study following aspects:

- Which connectivity model is better from security and privacy point of view?

  - How to enhance these connectivity models to ensure security and privacy?

- How to secure the connection for access to local services for the Femto?

### 5.X.2	Security Threats

If the appropriate connectivity model which ensures higher security and privacy is not selected, local access can lead to loss of confidentiality, integrity and/or availability.

### 5.X.3	Potential Requirements

The 5G system shall ensure that the connectivity model(s) used for local access is(are) secure and do not leak privacy details to any intruder.

******** END OF CHANGES ********