3GPP TSG-SA3 Meeting #118	S3-244131

Hyderabad, India 14 - 18 October 2024

# Source:	Qualcomm Incorporated

# Title:	Discussion on possible response to LTM LS

# Document for:	Discussion

Agenda Item:	4.12

# 1	Decision/action requested

This contribution discusses a possible response to the LS on LTM

# 2	References

[1] 	R2-2404037

# 3	Rationale

A quick opening remark for this contribution, it is written on the assumption (that the state of discussion in SA3 has not changed. Any changes will need to be reflected in any agreeable LS.

In their LS [1], RAN2 asks SA3 to consider some possible solutions for inter-CU LTM handovers. The LS contains high level descriptions of several possible (but not a complete set of possible) solutions.

SA3 have started to study possible solutions as part of completing the normative work on the inter CU handovers. The solutions included go beyond the ones contained in the RAN2 LS. The various possible solutions under discussion in SA3 that contain various amounts of signalling complexity. Such complexity is not really in scope of SA3 to analyse and decide between solutions as it is not in SA3’s remit to determine whether, for example, an RRC message or a MAC message adds the most complexity.

This may mean that the best way forward for this work is for SA3 to analyse the impacts of various ways of handling the keys and provide possible ways back to the RAN WGs along with the impact (and open issues in SA3 if any) and let RAN working groups decide on what is the best overall solution. This choice should then have a final review in SA3 to ensure that the security aims have been achieved. It would be good to inform RAN WGs of this possibility. RAN WGs may provide a clear guidance on the restrictions in developing solutions in SA3, e.g., downlink RRC signalling in subsequent inter-CU LTM is not allowed, so that SA3 can narrow the scope of potential solutions.

It is worth informing the RAN WGs of the following SA3 principles when studying solutions:

1.	If the PDCP anchor does not change then there is no need to rekey (i.e. the same principle that is applied to intra-CU LTM handovers and L3 handovers).

2.	When PDCP anchor changes, it is necessary to rekey. Further to this point, there is no agreement to relax the need to have at or provide to the target node a key unknown to the source anchor point for so-called forward security (despite SA3 discussing such solutions).

There are also some open discussions on the need for protection of a MAC CE, either to prevent potential DoS attacks or protect IEs added to these messages in various solutions. These issues are currently internal to SA3 and do not need to be liaised to the RAN WGs.

In their original LS, RAN2 asked the following:

RAN2 requests SA3 whether, for each option, the change of security algorithm or the change of key set indicator is to be supported for inter-CU LTM.

On the questions asked by RAN2 in the original LS, all handovers support the change of security algorithms and there seems no reason not to support this for inter-CU LTM handovers. On the change of key set indicator, our view is that the use of the change of key set indicator should be a rare event, so this does not need to be supported in LTM handovers provided it is possible to fall back to the existing handover procedure (e.g. L3) that allows this key to change to happen.

# 4	Detailed proposal

It is proposed to agree the response contained in S3-244132.